Revision History

| Revision | Date | Author | Reason for Change |
| --- | --- | --- | --- |
| 0.01 | 2016-03-15 | S5130S Team | COMWAREV700R001B70D001 First Draft, based on COMWARE V7 Platform MIB companion. |

# Contents

Revision History	i

About this document	3

Introduction	3

Consideration	3

Maintenance statement	5

Public MIBs list	5

Private MIBs list	7

Supported Standard Notifications	9

Supported Private Notifications	15

About this document

This document describes Management Information Base (MIB) and objects in them, which are supported by H3C products. It is supposed that this document gives a guideline for usage of MIBs in H3C products.

Introduction

This document contains “Public MIB List” section and “Private MIB list” section.

The “Public MIB List” provides a MIB list containing the Internet standard MIBs supported by products. The “Private MIB List” provides H3C private MIB list.

The following chapters describe the implementation of the objects of all public and private MIBs.

At the end of document, supported traps/notifications are presented.

Consideration

Whether all MIBs listed here are supported by products

H3C software platform provides a collection of MIBs to meet the needs of all series of products. However, a MIB is not necessarily supported even if it is included in a specific product software release. Some MIBs may work (The MIB objects can be read, or set from an NMS.), but they have not been verified or tested on any products.

If a MIB (or MIB object) is included in a software release but not in this document, this MIB (or MIB object) is not supported (not tested at least). If a MIB (or MIB object) can work, but is commented as “Not supported”, it is not supported (or not tested at least).

This document provides you with abundant MIBs. A MIB is supported only if the corresponding feature or function is supported. And if a MIB (table or scalar object) is not in this document, it represents that this MIB (table or scalar object) is not supported.

If a MIB is not supported or implemented, the notifications or traps (if any) of the MIB are not supported as well.

In consideration of compatibility with the lower versions and third party NMS implementation, some MIBs are deprecated and/or obsolete, but they cannot be removed from the software release.

MIBs and MIB objects may be updated (adding new ones or modification to the old ones). To get specific MIB information, refer to the newest private MIB structure and Internet Standard MIBs supported by H3C.

Difference between MIB and Command Line Interface (CLI)

Although you can perform configurations on the device through MIB, CLI is a basic way to make configurations on the device. In some cases, MIB operations and CLI operations have the same effect.

There are some limits on the usage of characters at the CLI. For example, in the following CLI, space is used as a delimiter to separate sysname and H3C.

[H3C] sysname H3C

Character ‘?’ is used as a help indicator in the CLI. If you type the character ‘?’ in the following CLI, a help message is displayed.

For example:

[H3C] sysname ?

STRING<1-30> System name (1 to 30 characters)

Because of the limitation of the length of the CLI, some configurations for “OCTET STRING” MIB object via SNMP may not be restored at the CLI. You are recommended to take the length limitation of CLI into account when executing configurations via MIB.

If the value of a MIB object in the set operation contains special characters, such as ‘?’ or space, you can perform the set operation via MIB, but the value of these MIB objects cannot be restored at the CLI, and information will be lost. Therefore, you are recommended to use visible characters (from 0x21 to 0x7E, except for 0x3F ‘?’) when performing MIB operations.

Table Format of MIB Information

In this document, scalar and tabular MIB object information is presented with the following table format. The Trap object excluded from such format of table.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ifIndex (1.3.6.1.2.1.2.2.1.1) | read-only | Yes | As per MIB |

The meanings of each column of the table are listed below.

| Item | Comments |
| --- | --- |
| ‘Name’ | Name of the MIB object |
| ‘Access’ | The ACCESS or MAX-ACCESS value as defined in MIBs. |
| ‘PDS’ | Permanent Data Storage. This field describes what will happen for MIB objects after device reboots. There are three statuses: 'Yes': After the value of a MIB object changed, the value of MIB object was saved automatically into device, the MIB object can remain the same value after device reboots. ‘No': whatever action is done, the value of MIB object will not remain after device reboots. Please note the ‘PDS’ of Counter/Counter32/Counter64 MIB object should be always ‘No’. ‘Current’: After the value of a MIB object changed, if the value of MIB object is saved explicitly (such as via ‘save’ CLI command), it can remain after device reboots. Otherwise, if not save explicitly, the value of MIB object will be lost. |
| ‘Description’ | General comments. All specific information are provided here. The following contents are just examples. ‘As per MIB’: Just as described in the MIB. ‘Not supported’: The object is not supported, although it can be operated (read and/or write). The user should not use such MIB object. ‘Need confirmed by products’: The information is product specific. Although information may provided in this field for some products, we cannot assure to list all product information. To find product specific information, users can access the device via SNMP. |

Maintenance statement

This document is used for MIBs supported by the platform software of H3C only. Differences between MIBs of different products must be provided. This document should not be directly delivered to end users. Relevant departments must adjust the contents of this document to fit in its requirement before delivering it to such end users.

Public MIBs list

In this section, all public MIBs supported by platform are listed.

Any action to modify, delete, remove and add must be recorded in Revision History

In File name column, the MIB file name must follow by the naming rule, which is the MIB module name prefixed with the RFC index like rfc4133-entity.mib. Note that file name is lowercase.

In MIB Version column, it indicates the MIB version used in devices.

| Module Name | File Name | comments | MIB Version |
| --- | --- | --- | --- |
| BGP4-MIB | rfc4273-bgp4.mib | Not supported | RFC4273 |
| BRIDGE-MIB | rfc1493-bridge.mib |  | RFC1493 |
| DISMAN-EVENT-MIB | rfc2981-disman-event.mib | Not supported | RFC2981 |
| DISMAN-PING-MIB | rfc2925-disman-ping.mib |  | RFC2925 |
| DISMAN-TRACEROUTE-MIB | rfc4560-disman-traceroute.mib |  | RFC4560 |
| DOT3-OAM-MIB | rfc4878-dot3-oam.mib |  | RFC4878 |
| ENTITY-MIB | rfc4133-entity.mib |  | RFC4133 |
| EtherLike-MIB | rfc3635-EtherLike.mib |  | RFC3635 |
| IEEE8021-CFM-MIB | ieee8021-cfm.mib |  | IEEEP802.1ag |
| IEEE8021-CFM-V2-MIB | ieee8021-cfm-v2.mib |  | IEEE802.1Q |
| IEEE8021-CN-MIB | ieee8021-cn.mib |  | IEEE P802.1Q |
| IEEE8023-LAG-MIB | ieee8023-lag.mib |  | IEEE8023 |
| IF-MIB | rfc2863-if.mib |  | RFC2863 |
| IGMP-STD-MIB | rfc2933-igmp-std.mib |  | RFC2933 |
| IP-MIB | rfc4293-ip.mib |  | RFC4293 |
| IP-FORWARD-MIB | rfc4292-ip-forward.mib |  | RFC4292 |
| IPMCAST-MIB | rfc5132-ipmcast.mib | Not supported | RFC5132 |
| IPV6-MIB | rfc2465-ipv6.mib |  | RFC2465 |
| IPV6-MLD-MIB | rfc3019-ipv6-mld.mib |  | RFC3019 |
| IPV6-ICMP-MIB | rfc2466-ipv6-icmp.mib |  | RFC2466 |
| IPV6-TCP-MIB | rfc2452-ipv6-tcp.mib |  | RFC2452 |
| IPV6-UDP-MIB | rfc2454-ipv6-udp.mib |  | RFC2454 |
| ISIS-MIB | rfc4444-isis.mib | Not supported | RFC4444 |
| LLDP-EXT-DOT1-EVB-EXTENSIONS-MIB | lldp-ext-dot1-evb-extensions.mib |  | IEEEStd 802.1Qbg-d2.2 |
| LLDP-EXT-DOT1-MIB | lldp-ext-dot1.mib |  | IEEE 802.1AB-2005 |
| LLDP-EXT-DOT1-V2-MIB | lldp-ext-dot1-v2.mib |  | IEEE 802.1AB-2009 |
| LLDP-EXT-DOT3-MIB | lldp-ext-dot3.mib |  | IEEE 802.1AB-2005 |
| LLDP-EXT-DOT3-V2-MIB | lldp-ext-dot3-v2.mib |  | IEEE 802.1AB-2009 |
| LLDP-EXT-MED-MIB | lldp-ext-med.mib |  | TIA-1057-2006 |
| LLDP-MIB | lldp.mib |  | IEEE 802.1AB-2005 |
| LLDP-V2-MIB | lldp-v2.mib |  | IEEE 802.1AB-2009 |
| MAU-MIB | rfc4836-mau.mib |  | RFC4836 |
| MGMD-STD-MIB | rfc5519-mgmd-std.mib | Not supported | RFC5519 |
| MPLS-FRR-FACILITY-STD-MIB | rfc6445-mpls-frr-facility-std.mib | Not supported | RFC6445 |
| MPLS-FRR-GENERAL-STD-MIB | rfc6445-mpls-frr-general-std.mib | Not supported | RFC6445 |
| MPLS-FTN-STD-MIB | rfc3814-mpls-ftn-std.mib | Not supported | RFC3814 |
| MPLS-L3VPN-STD-MIB | rfc4382-mpls-l3vpn-std.mib | Not supported | RFC4382 |
| MPLS-LDP-STD-MIB | rfc3815-mpls-ldp-std.mib | Not supported | RFC3815 |
| MPLS-LSR-STD-MIB | rfc3813-mpls-lsr-std.mib | Not supported | RFC3813 |
| MPLS-TE-STD-MIB | rfc3812-mpls-te-std.mib | Not supported | RFC3812 |
| OSPF-MIB | rfc4750-ospf.mib | Not supported | RFC4750 |
| OSPF-TRAP-MIB | rfc4750-ospf-trap.mib | Not supported | RFC4750 |
| OSPFV3-MIB | rfc5643-ospfv3.mib | Not supported | RFC5643 |
| PIM-BSR-MIB | rfc5240-pim-bsr.mib | Not supported | RFC5240 |
| PIM-STD-MIB | rfc5060-pim-std.mib | Not supported | RFC5060 |
| POWER-ETHERNET-MIB | rfc3621-power-ethernet.mib |  | RFC3621 |
| PPP-IP-NCP-MIB | rfc1473-ppp-ip.mib | Not supported | RFC1473 |
| PPP-LCP-MIB | rfc1471-ppp-lcp.mib | Not supported | RFC1471 |
| PW-ENET-STD-MIB | rfc5603-pw-enet-std.mib | Not supported | RFC5603 |
| PW-MPLS-STD-MIB | rfc5602-pw-mpls-std.mib | Not supported | RFC5602 |
| PW-STD-MIB | rfc5601-pw-std.mib |  | RFC5601 |
| Q-BRIDGE-MIB | rfc2674-qbridge.mib |  | RFC4363 |
| RADIUS-ACC-CLIENT-MIB | rfc2620-radius-acc-client.mib |  | RFC2620 |
| RADIUS-AUTH-CLIENT-MIB | rfc2618-radius-auth-client.mib |  | RFC2618 |
| RFC1213-MIB | rfc1213.mib |  | RFC1213 |
| RIPv2-MIB | rfc1724-rip.mib |  | RFC1724 |
| RMON-MIB | rfc2819-rmon.mib |  | RFC2819 |
| RMON2-MIB | rfc4502-rmon.mib |  | RFC4502 |
| SFLOW-MIB | sflow.mib |  | sFlow Version 5 |
| SNMP-FRAMEWORK-MIB | rfc2571-snmp-framework.mib |  | RFC2571 |
| SNMP-MPD-MIB | rfc2572-snmp-mpd.mib |  | RFC2572 |
| SNMP-NOTIFICATION-MIB | rfc3413-snmp-notification.mib |  | RFC3413 |
| SNMP-TARGET-MIB | rfc2573-snmp-target.mib |  | RFC2573 |
| SNMP-USER-BASED-SM-MIB | rfc3414-snmp-usm.mib |  | RFC3414 |
| SNMP-USM-AES-MIB | rfc3826-snmp-usm-aes.mib |  | RFC3826 |
| SNMP-VIEW-BASED-ACM-MIB | rfc3415-snmp-vacm.mib |  | RFC3415 |
| SNMPv2-TM | rfc3417-snmpv2-tm.mib | Not supported | RFC3417 |
| SNMPv2-MIB | rfc3418-snmpv2.mib |  | RFC3418 |
| TCP-MIB | rfc4022-tcp.mib |  | RFC4022 |
| TE-MIB | rfc3970-te.mib |  | RFC3970 |
| UDP-MIB | rfc4113-udp.mib |  | RFC4113 |
| VRRP-MIB | rfc2787-vrrp.mib | Not supported | RFC2787 |

Private MIBs list

| Module Name | File Name | comments | MIB Version |
| --- | --- | --- | --- |
| HH3C-ACL-MIB | hh3c-acl.mib |  | V3.3 |
| HH3C-BFD-STD-MIB | hh3c-bfd-std.mib |  | V1.1 |
| HH3C-CBQOS2-MIB | hh3c-cbqos2.mib |  | V1.8 |
| HH3C-COMMON-SYSTEM-MIB | hh3c-common-system.mib |  | V2.7 |
| HH3C-CONFIG-MAN-MIB | hh3c-config-man.mib |  | V2.6 |
| HH3C-DHCP-SNOOP2-MIB | hh3c-dhcp-snoop2.mib |  | V1.0 |
| HH3C-DHCP4-MIB | hh3c-dhcp4.mib | Not supported | V1.2 |
| HH3C-DLDP2-MIB | hh3c-dldp2.mib |  | V1.0 |
| HH3C-DNS-MIB | hh3c-dns.mib |  | V1.0 |
| HH3C-DOMAIN-MIB | hh3c-domain.mib |  | V1.8 |
| HH3C-ENTITY-EXT-MIB | hh3c-entity-ext.mib |  | V2.5 |
| HH3C-ENTRELATION-MIB | hh3c-entrelation.mib |  | V1.2 |
| HH3C-FLASH-MAN-MIB | hh3c-flash-man.mib |  | V3.3 |
| HH3C-IF-EXT-MIB | hh3c-if-ext.mib |  | V1.6 |
| HH3C-IFQOS2-MIB | hh3c-ifqos2.mib |  | V1.6 |
| HH3C-IKE-MONITOR-MIB | hh3c-ike-monitor.mib |  | V1.4 |
| HH3C-INFO-CENTER-MIB | hh3c-infocenter.mib |  | V1.0 |
| HH3C-IP-ADDRESS-MIB | hh3c-ip-address.mib |  | V1.5 |
| HH3C-IPRAN-DCN-MIB | hh3c-ipran-dcn.mib | Not supported | V1.0 |
| HH3C-IPSEC-MONITOR-V2-MIB | hh3c-ipsec-monitor-v2.mib | Not supported | V1.0 |
| HH3C-IPV6-ADDRESS-MIB | hh3c-ipv6-address.mib |  | V1.0 |
| HH3C-LAG-MIB | hh3c-lag.mib |  | V1.7 |
| HH3C-LBV2-MIB | hh3c-lbv2.mib | Not supported | V1.1 |
| HH3C-LICENSE-MIB | hh3c-license.mib |  | V1.0 |
| HH3C-LPBKDT-MIB | hh3c-lpbkdt.mib |  | V1.1 |
| HH3C-LSW-DEV-ADM-MIB | hh3c-lsw-dev-adm.mib |  | V3.66 |
| HH3C-LswDEVM-MIB | hh3c-splat-devm.mib |  | V2.2 |
| HH3C-LswIGSP-MIB | hh3c-splat-igsp.mib |  | V1.4 |
| HH3C-LswINF-MIB | hh3c-splat-inf.mib |  | V3.4 |
| HH3C-LswMAM-MIB | hh3c-splat-mam.mib |  | V2.3 |
| HH3C-LswMix-MIB | hh3c-splat-mix.mib |  | V1.2 |
| HH3C-MPLSEXT-MIB | hh3c-mplsext.mib | Not supported | V1.1 |
| HH3C-MPLSTE-MIB | hh3c-mplste.mib | Not supported | V1.0 |
| HH3C-LswVLAN-MIB | hh3c-splat-vlan.mib |  | V2.1 |
| HH3C-MAC-INFORMATION-MIB | hh3c-mac-information.mib |  | V1.1 |
| HH3C-MIRRORGROUP-MIB | hh3c-mirrorgroup.mib |  | V1.1 |
| HH3C-MPM-MIB | hh3c-mpm.mib |  | V1.2 |
| HH3C-NPV-MIB | hh3c-npv.mib |  | V1.0 |
| HH3C-NQA-MIB | hh3c-nqa.mib |  | V1.0 |
| HH3C-NTP-MIB | hh3c-ntp.mib |  | V1.7 |
| HH3C-NVGRE-MIB | hh3c-nvgre.mib | Not supported | V1.0 |
| HH3C-POWER-ETH-EXT-MIB | hh3c-power-eth-ext.mib |  | V1.5 |
| HH3C-PROTOCOL-VLAN-MIB | hh3c-protocol-vlan.mib |  | V1.4 |
| HH3C-QINQV2-MIB | hh3c-qinqv2.mib |  | V1.0 |
| HH3C-QOS-CAPABILITY-MIB | hh3c-qos-capability.mib |  | V1.2 |
| HH3C-RADIUS-MIB | hh3c-radius.mib |  | V2.3 |
| HH3C-RMON-EXT2-MIB | hh3c-rmon-ext2.mib |  | V1.0 |
| HH3C-SNMP-EXT-MIB | hh3c-snmp-ext.mib |  | V1.5 |
| HH3C-SSH-MIB | hh3c-ssh.mib |  | V1.3 |
| HH3C-STACK-MIB | hh3c-stack.mib |  | V1.0 |
| HH3C-STORM-CONSTRAIN-MIB | hh3c-storm-constrain.mib |  | V1.1 |
| HH3C-SUBNET-VLAN-MIB | hh3c-subnet-vlan.mib |  | V1.1 |
| HH3C-SYS-MAN-MIB | hh3c-sys-man.mib |  | V2.4 |
| HH3C-TRANSCEIVER-INFO-MIB | hh3c-transceiver-info.mib |  | V1.4 |
| HH3C-TRAP-MIB | hh3c-trap.mib |  | V1.5 |
| HH3C-TRNG2-MIB | hh3c-trng2.mib |  | V1.0 |
| HH3C-TUNNEL-MIB | hh3c-tunnel.mib | Not supported | V1.1 |
| HH3C-UI-MAN-MIB | hh3c-ui-man.mib |  | V1.6 |
| HH3C-USER-MIB | hh3c-user.mib |  | V2.6 |
| HH3C-VMAP-MIB | hh3c-vmap.mib |  | V1.0 |
| HH3C-VOICE-VLAN-MIB | hh3c-voice-vlan.mib |  | V1.3 |
| HH3C-VSAN-MIB | hh3c-vsan.mib |  | V1.1 |
| HH3C-VSI-MIB | hh3c-vsi.mib | Not supported | V1.5 |
| HH3C-VXLAN-MIB | hh3c-vxlan.mib | Not supported | V1.0 |

Supported Standard Notifications

This section contains a list of the traps/notification defined in the standard MIB (defined by IETF, IEEE or other standard organization).

| Notification | MIB | Description |
| --- | --- | --- |
| mplsXCUp (1.3.6.1.2.1.10.166.2.0.1) | MPLS-LSR-STD-MIB | This notification is generated when an mplsXCOperStatus object for one of the configured cross-connect entries is about to enter the up state from some other state. |
| mplsXCDown (1.3.6.1.2.1.10.166.2.0.2) | MPLS-LSR-STD-MIB | This notification is generated when an mplsXCOperStatus object for one of the configured cross-connect entries is about to enter the down state from some other state. |
| mplsTunnelUp (1.3.6.1.2.1.10.166.3.0.1) | MPLS-TE-STD-MIB | As per MIB |
| mplsTunnelDown (1.3.6.1.2.1.10.166.3.0.2) | MPLS-TE-STD-MIB | As per MIB |
| mplsTunnelRerouted (1.3.6.1.2.1.10.166.3.0.3) | MPLS-TE-STD-MIB | As per MIB |
| mplsTunnelReoptmized (1.3.6.1.2.1.10.166.3.0.4) | MPLS-TE-STD-MIB | As per MIB |
| mplsLdpInitSessionThresholdExceeded (1.3.6.1.2.1.10.166.4.0.1) | MPLS-LDP-STD-MIB | As per MIB |
| mplsLdpSessionUp (1.3.6.1.2.1.10.166.4.0.3) | MPLS-LDP-STD-MIB | As per MIB |
| mplsLdpSessionDown (1.3.6.1.2.1.10.166.4.0.4) | MPLS-LDP-STD-MIB | As per MIB |
| mplsL3VpnVrfUp (1.3.6.1.2.1.10.166.11.0.1) | MPLS-L3VPN-STD-MIB | As per MIB |
| mplsL3VpnVrfDown (1.3.6.1.2.1.10.166.11.0.2) | MPLS-L3VPN-STD-MIB | As per MIB |
| mplsL3VpnVrfRouteMidThreshExceeded (1.3.6.1.2.1.10.166.11.0.3) | MPLS-L3VPN-STD-MIB | As per MIB |
| mplsL3VpnVrfNumVrfRouteMaxThreshExceeded (1.3.6.1.2.1.10.166.11.0.4) | MPLS-L3VPN-STD-MIB | As per MIB |
| mplsL3VpnNumVrfSecIllglLblThrshExcd (1.3.6.1.2.1.10.166.11.0.5) | MPLS-L3VPN-STD-MIB | Not supported |
| mplsL3VpnNumVrfRouteMaxThreshCleared (1.3.6.1.2.1.10.166.11.0.6) | MPLS-L3VPN-STD-MIB | As per MIB |
| pwDown(1.3.6.1.2.1.10.246.0.1) | PW-STD-MIB | As per MIB |
| pwUp(1.3.6.1.2.1.10.246.0.2) | PW-STD-MIB | As per MIB |
| pwDeleted(1.3.6.1.2.1.10.246.0.3) | PW-STD-MIB | As per MIB |
| ospfVirtIfStateChange(1.3.6.1.2.1.14.16.2.1) | OSPF-TRAP-MIB | As per MIB |
| ospfNbrStateChange(1.3.6.1.2.1.14.16.2.2) | OSPF-TRAP-MIB | As per MIB |
| ospfVirtNbrStateChange(1.3.6.1.2.1.14.16.2.3) | OSPF-TRAP-MIB | As per MIB |
| ospfIfConfigError(1.3.6.1.2.1.14.16.2.4) | OSPF-TRAP-MIB | As per MIB |
| ospfVirtIfConfigError(1.3.6.1.2.1.14.16.2.5) | OSPF-TRAP-MIB | As per MIB |
| ospfIfAuthFailure(1.3.6.1.2.1.14.16.2.6) | OSPF-TRAP-MIB | As per MIB |
| ospfVirtIfAuthFailure(1.3.6.1.2.1.14.16.2.7) | OSPF-TRAP-MIB | As per MIB |
| ospfIfRxBadPacket(1.3.6.1.2.1.14.16.2.8) | OSPF-TRAP-MIB | As per MIB |
| ospfVirtIfRxBadPacket(1.3.6.1.2.1.14.16.2.9) | OSPF-TRAP-MIB | As per MIB |
| ospfTxRetransmit(1.3.6.1.2.1.14.16.2.10) | OSPF-TRAP-MIB | As per MIB |
| ospfVirtIfTxRetransmit(1.3.6.1.2.1.14.16.2.11) | OSPF-TRAP-MIB | As per MIB |
| ospfOriginateLsa(1.3.6.1.2.1.14.16.2.12) | OSPF-TRAP-MIB | As per MIB |
| ospfMaxAgeLsa(1.3.6.1.2.1.14.16.2.13) | OSPF-TRAP-MIB | As per MIB |
| ospfLsdbOverflow(1.3.6.1.2.1.14.16.2.14) | OSPF-TRAP-MIB | As per MIB |
| ospfLsdbApproachingOverflow(1.3.6.1.2.1.14.16.2.14) | OSPF-TRAP-MIB | As per MIB |
| ospfIfStateChange(1.3.6.1.2.1.14.16.2.16) | OSPF-TRAP-MIB | As per MIB |
| ospfNssaTranslatorStatusChange(1.3.6.1.2.1.14.16.2.17) | OSPF-TRAP-MIB | As per MIB |
| ospfRestartStatusChange(1.3.6.1.2.1.14.16.2.18) | OSPF-TRAP-MIB | As per MIB |
| ospfNbrRestartHelperStatusChange(1.3.6.1.2.1.14.16.2.19) | OSPF-TRAP-MIB | As per MIB |
| ospfVirtNbrRestartHelperStatusChange(1.3.6.1.2.1.14.16.2.20) | OSPF-TRAP-MIB | As per MIB |
| bgpEstablished (1.3.6.1.2.1.15.7.1) | BGP4-MIB | As per MIB |
| bgpBackwardTransition (1.3.6.1.2.1.15.7.2) | BGP4-MIB | As per MIB |
| bgpEstablishedNotification (1.3.6.1.2.1.15.0.1) | BGP4-MIB | As per MIB |
| bgpBackwardTransNotification (1.3.6.1.2.1.15.0.2) | BGP4-MIB | As per MIB |
| risingAlarm (1.3.6.1.2.1.16.0.1) | RMON-MIB | As per MIB |
| fallingAlarm (1.3.6.1.2.1.16.0.2) | RMON-MIB | As per MIB |
| entConfigChange(1.3.6.1.2.1.47.2.0.1) | ENTITY-MIB | As per MIB |
| ipv6IfStateChange (1.3.6.1.2.1.55.2.0.1) | IPV6-MIB | As per MIB |
| vrrpTrapNewMaster (1.3.6.1.2.1.68.0.1) | VRRP-MIB | As per MIB |
| vrrpTrapAuthFailure (1.3.6.1.2.1.68.0.2) | VRRP-MIB | As per MIB |
| pingProbeFailed (1.3.6.1.2.1.80.0.1) | DISMAN-PING-MIB | As per MIB |
| pingTestFailed (1.3.6.1.2.1.80.0.2) | DISMAN-PING-MIB | As per MIB |
| pingTestCompleted (1.3.6.1.2.1.80.0.3) | DISMAN-PING-MIB | As per MIB |
| traceRoutePathChange (1.3.6.1.2.1.81.0.1) | DISMAN-TRACEROUTE-MIB | As per MIB |
| traceRouteTestFailed (1.3.6.1.2.1.81.0.2) | DISMAN-TRACEROUTE-MIB | As per MIB |
| traceRouteTestCompleted (1.3.6.1.2.1.81.0.3) | DISMAN-TRACEROUTE-MIB | As per MIB |
| mteTriggerFired (1.3.6.1.2.1.88.2.0.1) | DISMAN-EVENT-MIB | As per MIB |
| mteTriggerRising (1.3.6.1.2.1.88.2.0.2) | DISMAN-EVENT-MIB | As per MIB |
| mteTriggerFalling (1.3.6.1.2.1.88.2.0.3) | DISMAN-EVENT-MIB | As per MIB |
| mteTriggerFailure (1.3.6.1.2.1.88.2.0.4) | DISMAN-EVENT-MIB | As per MIB |
| mteEventSetFailure (1.3.6.1.2.1.88.2.0.5) | DISMAN-EVENT-MIB | As per MIB |
| teTunnelUp (1.3.6.1.2.1.122.0.1) | TE-MIB | As per MIB |
| teTunnelDown (1.3.6.1.2.1.122.0.2) | TE-MIB | As per MIB |
| teTunnelChanged (1.3.6.1.2.1.122.0.3) | TE-MIB | As per MIB |
| isisDatabaseOverload (1.3.6.1.2.1.138.0.1) | ISIS-MIB | As per MIB |
| isisManualAddressDrops (1.3.6.1.2.1.138.0.2) | ISIS-MIB | As per MIB |
| isisCorruptedLSPDetected(1.3.6.1.2.1.138.0.3) | ISIS-MIB | As per MIB |
| isisAttemptToExceedMaxSequence(1.3.6.1.2.1.138.0.4) | ISIS-MIB | As per MIB |
| isisIDLenMismatch (1.3.6.1.2.1.138.0.5) | ISIS-MIB | As per MIB |
| isisMaxAreaAddressesMismatch (1.3.6.1.2.1.138.0.6) | ISIS-MIB | As per MIB |
| isisOwnLSPPurge (1.3.6.1.2.1.138.0.7) | ISIS-MIB | As per MIB |
| isisSequenceNumberSkip (1.3.6.1.2.1.138.0.8) | ISIS-MIB | As per MIB |
| isisAuthenticationTypeFailure (1.3.6.1.2.1.138.0.9) | ISIS-MIB | As per MIB |
| isisAuthenticationFailure (1.3.6.1.2.1.138.0.10) | ISIS-MIB | As per MIB |
| isisVersionSkew (1.3.6.1.2.1.138.0.11) | ISIS-MIB | As per MIB |
| isisAreaMismatch (1.3.6.1.2.1.138.0.12) | ISIS-MIB | As per MIB |
| isisRejectedAdjacency (1.3.6.1.2.1.138.0.13) | ISIS-MIB | As per MIB |
| isisLSPTooLargeToPropagate(1.3.6.1.2.1.138.0.14) | ISIS-MIB | As per MIB |
| isisOrigLSPBuffSizeMismatch(1.3.6.1.2.1.138.0.15) | ISIS-MIB | As per MIB |
| isisProtocolsSupportedMismatch(1.3.6.1.2.1.138.0.16) | ISIS-MIB | As per MIB |
| isisAdjacencyChange (1.3.6.1.2.1.138.0.17) | ISIS-MIB | As per MIB |
| isisLSPErrorDetected (1.3.6.1.2.1.138.0.18) | ISIS-MIB | As per MIB |
| pimNeighborLoss (1.3.6.1.2.1.157.0.1) | PIM-STD-MIB | As per MIB |
| pimBsrElectedBSRLostElection (1.3.6.1.2.1.172.0.1) | PIM-BSR-MIB | As per MIB |
| pimBsrCandidateBSRWinElection (1.3.6.1.2.1.172.0.2) | PIM-BSR-MIB | As per MIB |
| ospfv3VirtIfStateChange(1.3.6.1.2.1.191.0.1) | OSPFV3-MIB | As per MIB |
| ospfv3NbrStateChange(1.3.6.1.2.1.191.0.2) | OSPFV3-MIB | As per MIB |
| ospfv3VirtNbrStateChange(1.3.6.1.2.1.191.0.3) | OSPFV3-MIB | As per MIB |
| ospfv3IfConfigError(1.3.6.1.2.1.191.0.4) | OSPFV3-MIB | As per MIB |
| ospfv3VirtIfConfigError(1.3.6.1.2.1.191.0.5) | OSPFV3-MIB | As per MIB |
| ospfv3IfRxBadPacket(1.3.6.1.2.1.191.0.6) | OSPFV3-MIB | As per MIB |
| ospfv3VirtIfRxBadPacket(1.3.6.1.2.1.191.0.7) | OSPFV3-MIB | As per MIB |
| ospfv3LsdbOverflow(1.3.6.1.2.1.191.0.8) | OSPFV3-MIB | As per MIB |
| ospfv3LsdbApproachingOverflow(1.3.6.1.2.1.191.0.9) | OSPFV3-MIB | As per MIB |
| ospfv3IfStateChange(1.3.6.1.2.1.191.0.10) | OSPFV3-MIB | As per MIB |
| ospfv3NssaTranslatorStatusChange(1.3.6.1.2.1.191.0.11) | OSPFV3-MIB | As per MIB |
| ospfv3RestartStatusChange(1.3.6.1.2.1.191.0.12) | OSPFV3-MIB | As per MIB |
| ospfv3NbrRestartHelperStatusChange(1.3.6.1.2.1.191.0.13) | OSPFV3-MIB | As per MIB |
| ospfv3VirtNbrRestartHelperStatusChange(1.3.6.1.2.1.191.0.14) | OSPFV3-MIB | As per MIB |
| mplsFrrFacilityInitialBackupTunnelInvoked (1.3.6.1.2.1.204.0.1) | MPLS-FRR-FACILITY-STD-MIB | As per MIB |
| mplsFrrFacilityFinalTunnelRestored (1.3.6.1.2.1.204.0.2) | MPLS-FRR-FACILITY-STD-MIB | As per MIB |
| coldStart (1.3.6.1.6.3.1.1.5.1) | SNMPv2-MIB | As per MIB |
| warmStart (1.3.6.1.6.3.1.1.5.2) | SNMPv2-MIB | As per MIB |
| linkDown (1.3.6.1.6.3.1.1.5.3) | IF-MIB | The varbinds as defined in RFC1573. This will be used for VPN tunnel interfaces as well. In the case of dynamic VPNs (auto-mesh) - the ifIndex in the linkDown trap may no longer exist in the ifTable. |
| linkUp (1.3.6.1.6.3.1.1.5.4) | IF-MIB | The varbinds as defined in RFC1573. This will be used for VPN tunnel interfaces as well. In the case of dynamic VPNs (auto-mesh) - the ifIndex in the linkUp trap may no longer exist in the ifTable. |
| authenticationFailure (1.3.6.1.6.3.1.1.5.5) | SNMPv2-MIB | As per MIB |

Supported Private Notifications

This section contains a list of the traps/notifications defined in the private MIB (defined by enterprise).

| Trap | MIB | Description |
| --- | --- | --- |
| hh3cLogIn (1.3.6.1.4.1.25506.2.2.1.1.3.0.1) | HH3C-UI-MAN-MIB | As per MIB |
| hh3cLogOut (1.3.6.1.4.1.25506.2.2.1.1.3.0.2) | HH3C-UI-MAN-MIB | As per MIB |
| hh3cCfgManEventlog (1.3.6.1.4.1.25506.2.4.2.1) | HH3C-CONFIG-MAN-MIB | As per MIB |
| hh3cCfgOperateCompletion (1.3.6.1.4.1.25506.2.4.2.2) | HH3C-CONFIG-MAN-MIB | As per MIB |
| hh3cCfgInvalidConfigFile (1.3.6.1.4.1.25506.2.4.2.3) | HH3C-CONFIG-MAN-MIB | As per MIB |
| hh3cFlhOperNotification (1.3.6.1.4.1.25506.2.5.1.3.1) | HH3C-FLASH-MAN-MIB | As per MIB |
| hh3cEntityExtTemperatureThresholdNotification (1.3.6.1.4.1.25506.2.6.2.0.1) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtVoltageLowThresholdNotification (1.3.6.1.4.1.25506.2.6.2.0.2) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtVoltageHighThresholdNotification (1.3.6.1.4.1.25506.2.6.2.0.3) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtCpuUsageThresholdNotfication (1.3.6.1.4.1.25506.2.6.2.0.4) | HH3C-ENTITY-EXT-MIB | The notification is sent every 60 seconds until the CPU usage of the module goes down below the upper limit. |
| hh3cEntityExtMemUsageThresholdNotification (1.3.6.1.4.1.25506.2.6.2.0.5) | HH3C-ENTITY-EXT-MIB | The notification is sent every 60 seconds until the memory usage of the module goes down below the upper limit. |
| hh3cEntityExtOperEnabled (1.3.6.1.4.1.25506.2.6.2.0.6) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtOperDisabled (1.3.6.1.4.1.25506.2.6.2.0.7) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtCriticalTemperatureThresholdNotification (1.3.6.1.4.1.25506.2.6.2.0.8) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtSFPAlarmOn (1.3.6.1.4.1.25506.2.6.2.0.9) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtSFPAlarmOff (1.3.6.1.4.1.25506.2.6.2.0.10) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtSFPPhony (1.3.6.1.4.1.25506.2.6.2.0.11) | HH3C-ENTITY-EXT-MIB | This module is NOT sold by H3C. H3C therefore shall NOT guarantee the normal function of the device or assume the maintenance responsibility thereof. The notification is generated periodically after a phony module has been found. |
| hh3cEntityInsert (1.3.6.1.4.1.25506.2.6.2.0.12) | HH3C-ENTITY-EXT-MIB | As per MIB |
| hh3cEntityRemove (1.3.6.1.4.1.25506.2.6.2.0.13) | HH3C-ENTITY-EXT-MIB | As per MIB |
| hh3cEntityExtForcedPowerOff (1.3.6.1.4.1.25506.2.6.2.0.14) | HH3C-ENTITY-EXT-MIB | As per MIB |
| hh3cEntityExtForcedPowerOn (1.3.6.1.4.1.25506.2.6.2.0.15) | HH3C-ENTITY-EXT-MIB | As per MIB |
| hh3cEntityExtFaultAlarmOn (1.3.6.1.4.1.25506.2.6.2.0.16) | HH3C-ENTITY-EXT-MIB | As per MIB |
| hh3cEntityExtFaultAlarmOff (1.3.6.1.4.1.25506.2.6.2.0.17) | HH3C-ENTITY-EXT-MIB | As per MIB |
| hh3cEntityExtResourceLack(1.3.6.1.4.1.25506.2.6.2.0.18) | HH3C-ENTITY-EXT-MIB | As per MIB |
| hh3cEntityExtResourceEnough(1.3.6.1.4.1.25506.2.6.2.0.19) | HH3C-ENTITY-EXT-MIB | As per MIB |
| hh3cEntityExtTemperatureLower (1.3.6.1.4.1.25506.2.6.2.0.20) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtTemperatureTooUp (1.3.6.1.4.1.25506.2.6.2.0.21) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtTemperatureNormal (1.3.6.1.4.1.25506.2.6.2.0.22) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExternalAlarmOccur (1.3.6.1.4.1.25506.2.6.2.0.23) | HH3C-ENTITY-EXT-MIB | The notification is generaged when the monitored device connected to the specified entity fails. |
| hh3cEntityExternalAlarmRecover (1.3.6.1.4.1.25506.2.6.2.0.24) | HH3C-ENTITY-EXT-MIB | The notification is generated when the failed device connected to the specified entity retruns to normal. |
| hh3cEntityExtCpuUsageThresholdRecover (1.3.6.1.4.1.25506.2.6.2.0.25) | HH3C-ENTITY-EXT-MIB | The notification indicates the CPU usage descends the threshold. |
| hh3cEntityExtMemUsageThresholdRecover (1.3.6.1.4.1.25506.2.6.2.0.26) | HH3C-ENTITY-EXT-MIB | The notification indicates the memory usage descends the threshold. |
| hh3cEntityExtCritLowerTempThresholdNotification(1.3.6.1.4.1.25506.2.6.2.0.29) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cEntityExtTemperatureTooLow(1.3.6.1.4.1.25506.2.6.2.0.30) | HH3C-ENTITY-EXT-MIB | Not supported |
| hh3cAclRuleMatchCount (1.3.6.1.4.1.25506.2.8.5.0.2) | HH3C-ACL-MIB | As per MIB |
| hh3cAclFirstIPv4PktCaptured(1.3.6.1.4.1.25506.2.8.5.0.3) | HH3C-ACL-MIB | Not supported |
| hh3cAclFirstIPv6PktCaptured(1.3.6.1.4.1.25506.2.8.5.0.4) | HH3C-ACL-MIB | Not supported |
| hh3cAclFirstEthernetPktCaptured(1.3.6.1.4.1.25506.2.8.5.0.5) | HH3C-ACL-MIB | Not supported |
| hh3cIKETunnelStart (1.3.6.1.4.1.25506.2.30.1.6.1.1) | HH3C-IKE-MONITOR-MIB | As per MIB |
| hh3cIKETunnelStop (1.3.6.1.4.1.25506.2.30.1.6.1.2) | HH3C-IKE-MONITOR-MIB | As per MIB |
| hh3cIKENoSaFailure (1.3.6.1.4.1.25506.2.30.1.6.1.3) | HH3C-IKE-MONITOR-MIB | As per MIB |
| hh3cIKEEncryFailFailure (1.3.6.1.4.1.25506.2.30.1.6.1.4) | HH3C-IKE-MONITOR-MIB | As per MIB |
| hh3cIKEDecryFailFailure (1.3.6.1.4.1.25506.2.30.1.6.1.5) | HH3C-IKE-MONITOR-MIB | As per MIB |
| hh3cIKEInvalidProposalFailure (1.3.6.1.4.1.25506.2.30.1.6.1.6) | HH3C-IKE-MONITOR-MIB | As per MIB |
| hh3cIKEAuthFailFailure (1.3.6.1.4.1.25506.2.30.1.6.1.7) | HH3C-IKE-MONITOR-MIB | As per MIB |
| hh3cIKEInvalidCookieFailure (1.3.6.1.4.1.25506.2.30.1.6.1.8) | HH3C-IKE-MONITOR-MIB | As per MIB |
| hh3cIKEAttrNotSuppFailure (1.3.6.1.4.1.25506.2.30.1.6.1.9) | HH3C-IKE-MONITOR-MIB | Not supported |
| hh3cIKEUnsportExchTypeFailure (1.3.6.1.4.1.25506.2.30.1.6.1.10) | HH3C-IKE-MONITOR-MIB | As per MIB |
| hh3cIKEInvalidIdFailure (1.3.6.1.4.1.25506.2.30.1.6.1.11) | HH3C-IKE-MONITOR-MIB | The object hh3cIKEIdInformation is not supported, will always return empty string. |
| hh3cIKEInvalidProtocolFailure (1.3.6.1.4.1.25506.2.30.1.6.1.12) | HH3C-IKE-MONITOR-MIB | The object hh3cIKEProtocolNum is not supported, will always return 0. |
| hh3cIKECertTypeUnsuppFailure (1.3.6.1.4.1.25506.2.30.1.6.1.13) | HH3C-IKE-MONITOR-MIB | Not supported |
| hh3cIKEInvalidCertAuthFailure (1.3.6.1.4.1.25506.2.30.1.6.1.14) | HH3C-IKE-MONITOR-MIB | Not supported |
| hh3cIKEInvalidSignFailure (1.3.6.1.4.1.25506.2.30.1.6.1.15) | HH3C-IKE-MONITOR-MIB | Not supported |
| hh3cIKECertUnavailableFailure (1.3.6.1.4.1.25506.2.30.1.6.1.16) | HH3C-IKE-MONITOR-MIB | The object hh3cIKECertInformation is not supported, will always return empty string. |
| hh3cIKEProposalAdd (1.3.6.1.4.1.25506.2.30.1.6.1.17) | HH3C-IKE-MONITOR-MIB | As per MIB |
| hh3cIKEProposalDel (1.3.6.1.4.1.25506.2.30.1.6.1.18) | HH3C-IKE-MONITOR-MIB | As per MIB |
| hh3cRtTabFullTrap (1.3.6.1.4.1.25506.2.38.1.3.5.1) | HH3C-TRAP-MIB | Send this trap when the routing table is filled. The interval between two traps generated should be longer than hh3cRtTabTrapInterval. |
| hh3cPeriodicalTrap (1.3.6.1.4.1.25506.2.38.1.6.3.0.1) | HH3C-TRAP-MIB | Not supported |
| hh3cIpAddressChangeNotify (1.3.6.1.4.1.25506.2.67.2.2.0.1) | HH3C-IP-ADDRESS-MIB | As per MIB |
| hh3cBfdSessStateUp (1.3.6.1.4.1.25506.2.72.0.3) | HH3C-BFD-STD-MIB | As per MIB |
| hh3cBfdSessStateDown(1.3.6.1.4.1.25506.2.72.0.4) | HH3C-BFD-STD-MIB | As per MIB |
| hh3cBfdSessReachLimit ( 1.3.6.1.4.1.25506.2.72.0.5) | HH3C-BFD-STD-MIB | As per MIB |
| hh3cRmonExtRisingAlarm (1.3.6.1.4.1.25506.2.125.0.1) | HH3C-RMON-EXT2-MIB | As per MIB |
| hh3cRmonExtFallingAlarm (1.3.6.1.4.1.25506.2.125.0.2) | HH3C-RMON-EXT2-MIB | As per MIB |
| hh3cIPsecTunnelStartV2 (1.3.6.1.4.1.25506.2.126.1.9.0.1) | HH3C-IPSEC-MONITOR-V2-MIB | As per MIB |
| hh3cIPsecTunnelStopV2 (1.3.6.1.4.1.25506.2.126.1.9.0.2) | HH3C-IPSEC-MONITOR-V2-MIB | As per MIB |
| hh3cIPsecNoSaFailureV2 (1.3.6.1.4.1.25506.2.126.1.9.0.3) | HH3C-IPSEC-MONITOR-V2-MIB | As per MIB |
| hh3cIPsecAuthFailFailureV2 (1.3.6.1.4.1.25506.2.126.1.9.0.4) | HH3C-IPSEC-MONITOR-V2-MIB | As per MIB |
| hh3cIPsecEncryFailFailureV2 (1.3.6.1.4.1.25506.2.126.1.9.0.5) | HH3C-IPSEC-MONITOR-V2-MIB | As per MIB |
| hh3cIPsecDecryFailFailureV2 (1.3.6.1.4.1.25506.2.126.1.9.0.6) | HH3C-IPSEC-MONITOR-V2-MIB | As per MIB |
| hh3cIPsecInvalidSaFailureV2 (1.3.6.1.4.1.25506.2.126.1.9.0.7) | HH3C-IPSEC-MONITOR-V2-MIB | Not supported |
| hh3cIPsecPolicyAddV2 (1.3.6.1.4.1.25506.2.126.1.9.0.8) | HH3C-IPSEC-MONITOR-V2-MIB | As per MIB |
| hh3cIPsecPolicyDelV2 (1.3.6.1.4.1.25506.2.126.1.9.0.9) | HH3C-IPSEC-MONITOR-V2-MIB | As per MIB |
| hh3cIPsecPolicyAttachV2 (1.3.6.1.4.1.25506.2.126.1.9.0.10) | HH3C-IPSEC-MONITOR-V2-MIB | As per MIB |
| hh3cIPsecPolicyDetachV2 (1.3.6.1.4.1.25506.2.126.1.9.0.11) | HH3C-IPSEC-MONITOR-V2-MIB | As per MIB |
| hh3cLicenseOpCompletion (1.3.6.1.4.1.25506.2.145.3.0.1) | HH3C-LICENSE-MIB | As per MIB |
| hh3cLicenseActivationFileLost (1.3.6.1.4.1.25506.2.145.3.0.2) | HH3C-LICENSE-MIB | As per MIB |
| hh3cLicenseActivationFileRestored (1.3.6.1.4.1.25506.2.145.3.0.3) | HH3C-LICENSE-MIB | As per MIB |
| hh3cLicenseExpired (1.3.6.1.4.1.25506.2.145.3.0.4) | HH3C-LICENSE-MIB | As per MIB |
| hh3cLicenseExpireWarning (1.3.6.1.4.1.25506.2.145.3.0.5) | HH3C-LICENSE-MIB | As per MIB |
| hh3cLBv2VSConnsOverloadTrap(1.3.6.1.4.1.25506.2.148.6.0.1) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2VSConnRecoveryTrap(1.3.6.1.4.1.25506.2.148.6.0.2) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2VSConnsRateOverloadTrap(1.3.6.1.4.1.25506.2.148.6.0.3) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2VSConnsRateRecoveryTrap(1.3.6.1.4.1.25506.2.148.6.0.4) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2VSActiveTrap(1.3.6.1.4.1.25506.2.148.6.0.5) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2VSInactiveTrap(1.3.6.1.4.1.25506.2.148.6.0.6) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2RSAvailableTrap(1.3.6.1.4.1.25506.2.148.6.0.7) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2RSUnavailableTrap(1.3.6.1.4.1.25506.2.148.6.0.8) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2SFActiveTrap(1.3.6.1.4.1.25506.2.148.6.0.9) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2SFInactiveTrap(1.3.6.1.4.1.25506.2.148.6.0.10) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2ActionInUseSFChangeTrap(1.3.6.1.4.1.25506.2.148.6.0.11) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2VSInUseSFChangeTrap(1.3.6.1.4.1.25506.2.148.6.0.12) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2RSConnOverloadTrap(1.3.6.1.4.1.25506.2.148.6.0.13) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2RSConnRecoveryTrap(1.3.6.1.4.1.25506.2.148.6.0.14) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2RSConnsRateOverloadTrap(1.3.6.1.4.1.25506.2.148.6.0.15) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cLBv2RSConnsRateRecoveryTrap(1.3.6.1.4.1.25506.2.148.6.0.16) | HH3C-LBV2-MIB | hh3c-lbv2.mib |
| hh3cIpRanDcnNeOnline (1.3.6.1.4.1.25506.2.152.1.3.0.1) | HH3C-IPRAN-DCN-MIB | As per MIB |
| hh3cIpRanDcnNeOffline (1.3.6.1.4.1.25506.2.152.1.3.0.2) | HH3C-IPRAN-DCN-MIB | As per MIB |
| hh3cWriteSuccessTrap (1.3.6.1.4.1.25506.6.8.1) | HH3C-COMMON-SYSTEM-MIB | Not supported |
| hh3cWriteFailureTrap (1.3.6.1.4.1.25506.6.8.2) | HH3C-COMMON-SYSTEM-MIB | Not supported |
| hh3cRebootSendTrap (1.3.6.1.4.1.25506.6.8.3) | HH3C-COMMON-SYSTEM-MIB | As per MIB |
| hh3cSysColdStartTrap (1.3.6.1.4.1.25506.6.8.4) | HH3C-COMMON-SYSTEM-MIB | Not supported |
| hh3cSysWarmStartTrap (1.3.6.1.4.1.25506.6.8.5) | HH3C-COMMON-SYSTEM-MIB | Not supported |
| hh3cNqaProbeTimeOverThreshold(1.3.6.1.4.1.25506.8.3.3.1) | HH3C-NQA-MIB | As per MIB |
| hh3cNqaJitterRTTOverThreshold (1.3.6.1.4.1.25506.8.3.3.2) | HH3C-NQA-MIB | As per MIB |
| hh3cNqaProbeFailure (1.3.6.1.4.1.25506.8.3.3.3) | HH3C-NQA-MIB | As per MIB |
| hh3cNqaJitterPacketLoss (1.3.6.1.4.1.25506.8.3.3.4) | HH3C-NQA-MIB | As per MIB |
| hh3cNqaJitterSDOverThreshold (1.3.6.1.4.1.25506.8.3.3.5) | HH3C-NQA-MIB | As per MIB |
| hh3cNqaJitterDSOverThreshold (1.3.6.1.4.1.25506.8.3.3.6) | HH3C-NQA-MIB | As per MIB |
| hh3cNqaICPIFOverThreshold (1.3.6.1.4.1.25506.8.3.3.7) | HH3C-NQA-MIB | As per MIB |
| hh3cNqaMOSOverThreshold (1.3.6.1.4.1.25506.8.3.3.8) | HH3C-NQA-MIB | As per MIB |
| hh3cL2vpnPwSwitchPtoB (1.3.6.1.4.1.25506.2.162.0.1) | HH3C-L2VPN-MIB | As per MIB |
| hh3cL2vpnPwSwitchBtoP (1.3.6.1.4.1.25506.2.162.0.2) | HH3C-L2VPN-MIB | As per MIB |