# Contents

DISMAN-PING-MIB	2

Scalar objects	2

pingCtlTable	2

pingResultsTable	8

pingProbeHistoryTable	9

DISMAN-PING-MIB

This MIB should be supported by device which implements disman ping function. This MIB cannot mix using with CLI.

Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| pingMaxConcurrentRequests (1.3.6.1.2.1.80.1.1) | read-only | Current | As Per MIB |

pingCtlTable

OID of this table is: 1.3.6.1.2.1.80.1.2

  - The parameter which is not supported by the corresponding pingCtlType cannot be configured.

  - When creating a test entry, the default value of pingCtlType is pingIcmpEcho. Once created, pingCtlType cannot be changed, so pingCtlType must be set as the entry created.

  - When creating an entry, if the parameter which is not supported by the corresponding pingCtlType is set, the operation will be failed.

  - Any of the configurations in pingCtlTable changed, the corresponding results, histories and statistics will be cleared, except the following objects:

pingCtlTrapGeneration,

pingCtlTrapProbeFailureFilter,

pingCtlTrapTestFailureFilter,

pingCtlDescr,

pingCtlMaxRows(If the value of this object is smaller than the previous value, the redundant histories records will be deleted.)

The results, histories and statistics must be the factual reflection of the test results of current configurations.

Users will be puzzled when they get the results which caused by the previous configurations.

  - pingCtlTargetAddressType and pingCtlTargetAddress must be modified together.

  - pingCtlSourceAddressType and pingCtlSourceAddress must be modified together.

  - If the parameter which is not supported by the corresponding pingCtlType is read, the result is invalid.

The following objects are supported by all pingCtlTypes:

pingCtlOwnerIndex, pingCtlTestName, pingCtlTimeOut,

pingCtlAdminStatus, pingCtlFrequency,

pingCtlStorageType, pingCtlTrapGeneration, pingCtlType, pingCtlDescr, pingCtlRowStatus

The remainder objects is supported by part of pingCtlTypes.

The relationship between these objects and corresponding pingCtlType are described in the following table:

| Value of pingCtlType | Support parameters |
| --- | --- |
| pingIcmpEcho | pingCtlTargetAddressType pingCtlTargetAddress pingCtlDataSize pingCtlDataFill pingCtlSourceAddressType pingCtlSourceAddress pingCtlIfIndex pingCtlByPassRouteTable pingCtlDSField pingCtlProbeCount pingCtlMaxRows pingCtlTrapProbeFailureFilter pingCtlTrapTestFailureFilter |
| pingUdpEcho hh3cNqaUdpEcho or hh3cpingUdpEcho | pingCtlTargetAddressType pingCtlTargetAddress pingCtlDataSize pingCtlDataFill pingCtlSourceAddressType pingCtlSourceAddress pingCtlByPassRouteTable pingCtlDSField pingCtlProbeCount pingCtlMaxRows pingCtlTrapProbeFailureFilter pingCtlTrapTestFailureFilter |
| pingSnmpQuery | pingCtlTargetAddressType pingCtlTargetAddress pingCtlSourceAddressType pingCtlSourceAddress pingCtlByPassRouteTable pingCtlDSField pingCtlProbeCount pingCtlMaxRows pingCtlTrapProbeFailureFilter pingCtlTrapTestFailureFilter |
| pingTcpConnectionAttempt hh3cNqaTcpconnect or hh3cpingTcpconnect | pingCtlTargetAddressType pingCtlTargetAddress pingCtlSourceAddressType pingCtlSourceAddress pingCtlByPassRouteTable pingCtlDSField pingCtlProbeCount pingCtlMaxRows pingCtlTrapProbeFailureFilter pingCtlTrapTestFailureFilter |
| hh3cNqajitter ( for udp-jitter test) hh3cNqaCtlCodecType is defined as notDefined(1) | pingCtlTargetAddressType pingCtlTargetAddress pingCtlDataSize pingCtlDataFill pingCtlSourceAddressType pingCtlSourceAddress pingCtlByPassRouteTable pingCtlDSField pingCtlProbeCount |
| hh3cNqajitter (for voice test) hh3cNqaCtlCodecType is defined as g711Alaw(2)、g711Ulaw(3) or g729A(4) | pingCtlTargetAddressType pingCtlTargetAddress pingCtlDataSize pingCtlDataFill pingCtlSourceAddressType pingCtlSourceAddress pingCtlByPassRouteTable pingCtlDSField pingCtlProbeCount(only 1 is legal) |
| hh3cNqaHttp | pingCtlTargetAddressType pingCtlTargetAddress pingCtlSourceAddressType pingCtlSourceAddress pingCtlByPassRouteTable pingCtlDSField pingCtlProbeCount pingCtlMaxRows pingCtlTrapProbeFailureFilter pingCtlTrapTestFailureFilter |
| hh3cNqadlsw | pingCtlTargetAddressType pingCtlTargetAddress pingCtlSourceAddressType pingCtlSourceAddress pingCtlByPassRouteTable pingCtlDSField pingCtlProbeCount pingCtlMaxRows pingCtlTrapProbeFailureFilter pingCtlTrapTestFailureFilter |
| hh3cNqadhcp | pingCtlTargetAddressType pingCtlTargetAddress pingCtlSourceAddressType pingCtlSourceAddress pingCtlProbeCount pingCtlMaxRows pingCtlTrapProbeFailureFilter pingCtlTrapTestFailureFilter pingCtlIfIndex |
| hh3cNqaftp | pingCtlTargetAddressType pingCtlTargetAddress pingCtlSourceAddressType pingCtlSourceAddress pingCtlByPassRouteTable pingCtlDSField pingCtlProbeCount pingCtlMaxRows pingCtlTrapProbeFailureFilter pingCtlTrapTestFailureFilter |

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| pingCtlOwnerIndex (1.3.6.1.2.1.80.1.2.1.1) | not-accessible | Current | The uppercase letters in this string will be converted to lowercase letters in setting operation. The value of this object cannot contain ‘-‘. |
| pingCtlTestName (1.3.6.1.2.1.80.1.2.1.2) | not-accessible | Current | The uppercase letters in this string will be converted to lowercase letters in setting operation. The value of this object cannot contain ‘-‘. |
| pingCtlTargetAddressType (1.3.6.1.2.1.80.1.2.1.3) | read-create | Current | Only support ipv4(1) and unknown(0) and dns(16). |
| pingCtlTargetAddress (1.3.6.1.2.1.80.1.2.1.4) | read-create | Current | If the value of pingCtlTargetAddressType is ipv4(1), the value of this object must be ipv4 address. If the value of pingCtlTargetAddressType is dns(16),the value of this object can be host name. If the value of pingCtlTargetAddressType is unknown(0), the value of this object must be zero-length string. |
| pingCtlDataSize (1.3.6.1.2.1.80.1.2.1.5) | read-create | Current | An IP packet has a maximum size of 8100 octets, not including the size of the ICMP or UDP header (both 8 octets) and the size of the IP header (20 octets). 0 octets means system use the default packet size. When doing icmp or udp test, the range is from 20 to 8100. When doing JITTER test, the range is from68 to 8100 and the default value is 100. When doing voice test, the range is from 16 to 1500, and if the codec-type is g711a or g711u,the default value is 172, or else the default value is 32. |
| pingCtlTimeOut (1.3.6.1.2.1.80.1.2.1.6) | read-create | Current | If the value is not times of seconds(set by CLI), the result of reading operation is inaccurate. Such as: If the value set by CLI is 1050ms, the result of reading operation is 2s. If the value exceeds 60s(set by CLI), the result of reading operation is 60s. When doing voice test, default value is 5s,or else the default value is 3s |
| pingCtlProbeCount (1.3.6.1.2.1.80.1.2.1.7) | read-create | Current | For voice test, the value of this object only can set 1 |
| pingCtlAdminStatus (1.3.6.1.2.1.80.1.2.1.8) | read-create | Current | As per MIB |
| pingCtlDataFill (1.3.6.1.2.1.80.1.2.1.9) | read-create | Current | The length of this object is from 0 to 200. |
| pingCtlFrequency (1.3.6.1.2.1.80.1.2.1.10) | read-create | Current | Range from 0 to 604800 If the value is not times of seconds (set by CLI), the result of reading operation is inaccurate. Such as: If the value set by CLI is 1050ms, the result of reading operation is 2s. When doing voice test, default value is 60s,or else the default value is 0s. If the value of this object is 0, statistics will not be created. |
| pingCtlMaxRows (1.3.6.1.2.1.80.1.2.1.11) | read-create | Current | Range from 0 to 50. |
| pingCtlStorageType (1.3.6.1.2.1.80.1.2.1.12) | read-only | Current | As per MIB |
| pingCtlTrapGeneration (1.3.6.1.2.1.80.1.2.1.13) | read-create | Current | For voice and udp-jitter test, Only pingTestCompleted(2) is supported. |
| pingCtlTrapProbeFailureFilter (1.3.6.1.2.1.80.1.2.1.14) | read-create | Current | Range from 1 to 15. This object cannot be configured before the corresponding type(probeFailure) in pingCtlTrapGeneration is specified. If the value of pingCtlTrapGeneration does not contain probeFailure, the value of this object is 1. For voice test, this object is invalid. |
| pingCtlTrapTestFailureFilter (1.3.6.1.2.1.80.1.2.1.15) | read-create | Current | Range from 1 to 15. This object cannot be configured before the corresponding type(testFailure) in pingCtlTrapGeneration is specified. If the value of pingCtlTrapGeneration does not contain testFailure, the value of this object is 1. For voice test, this object is invalid. |
| pingCtlType (1.3.6.1.2.1.80.1.2.1.16) | read-create | Current | Can not be modified after creation. |
| pingCtlDescr (1.3.6.1.2.1.80.1.2.1.17) | read-create | Current | The length of this object is from 0 to 200. The value cannot begin with space. |
| pingCtlSourceAddressType (1.3.6.1.2.1.80.1.2.1.18) | read-create | Current | Only support ipv4(1) and unknown(0). Default value is unknown(0). |
| pingCtlSourceAddress (1.3.6.1.2.1.80.1.2.1.19) | read-create | Current | If the value of pingCtlSourceAddressType is ipv4(1), the value of this object must be ipv4 address. If the value of pingCtlSourceAddressType is unknown(0), the value of this object must be zero-length string. |
| pingCtlIfIndex (1.3.6.1.2.1.80.1.2.1.20) | read-create | Current | As per MIB |
| pingCtlByPassRouteTable (1.3.6.1.2.1.80.1.2.1.21) | read-create | Current | As per MIB |
| pingCtlDSField (1.3.6.1.2.1.80.1.2.1.22) | read-create | Current | As per MIB |
| pingCtlRowStatus (1.3.6.1.2.1.80.1.2.1.23) | read-create | No | Only support active(1), createAndgo(4) and destroy(6). |

pingResultsTable

OID of this table is: 1.3.6.1.2.1.80.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| pingResultsOperStatus (1.3.6.1.2.1.80.1.3.1.1) | read-only | No | As per MIB |
| pingResultsIpTargetAddressType (1.3.6.1.2.1.80.1.3.1.2) | read-only | No | As per MIB |
| pingResultsIpTargetAddress (1.3.6.1.2.1.80.1.3.1.3) | read-only | No | As per MIB |
| pingResultsMinRtt (1.3.6.1.2.1.80.1.3.1.4) | read-only | No | As per MIB |
| pingResultsMaxRtt (1.3.6.1.2.1.80.1.3.1.5) | read-only | No | As per MIB |
| pingResultsAverageRtt (1.3.6.1.2.1.80.1.3.1.6) | read-only | No | As per MIB |
| pingResultsProbeResponses (1.3.6.1.2.1.80.1.3.1.7) | read-only | No | As per MIB |
| pingResultsSentProbes (1.3.6.1.2.1.80.1.3.1.8) | read-only | No | As per MIB |
| pingResultsRttSumOfSquares (1.3.6.1.2.1.80.1.3.1.9) | read-only | No | As per MIB |
| pingResultsLastGoodProbe (1.3.6.1.2.1.80.1.3.1.10) | read-only | No | As per MIB |

pingProbeHistoryTable

OID of this table is: 1.3.6.1.2.1.80.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| pingProbeHistoryIndex (1.3.6.1.2.1.80.1.4.1.1) | not-accessible | No | As per MIB |
| pingProbeHistoryResponse (1.3.6.1.2.1.80.1.4.1.2) | read-only | No | As per MIB |
| pingProbeHistoryStatus (1.3.6.1.2.1.80.1.4.1.3) | read-only | No | The result of a particular probe done by a remote host. |
| pingProbeHistoryLastRC (1.3.6.1.2.1.80.1.4.1.4) | read-only | No | Not supported |
| pingProbeHistoryTime (1.3.6.1.2.1.80.1.4.1.5) | read-only | No | As per MIB |