# Contents

ENTITY-MIB	2

entPhysicalTable	3

entLogicalTable	4

entLPMappingTable	5

entAliasMappingTable	5

entPhysicalContainsTable	5

entityGeneral Group	5

- ENTITY-MIB

This MIB is supported only when the products support RFC4133.

This chapter describes the information of Entity MIB. The Entity MIB contains five groups of MIB objects: entityPhysical group, entityLogical group, entityMapping group, entityGeneral group and entityNotifications group. Thereinto, the entityPhysical group describes the physical entities managed by a single agent. This group contains a single table to identify physical system components, called the entPhysicalTable. The entPhysicalTable contains one row per physical entity, and must always contain at least one row for an "overall" physical entity, which should have an entPhysicalClass value of 'stack(11)', 'chassis(3)' or 'module(9)'. Each row is indexed by an arbitrary, small integer, and contains a description and type of the physical entity.  It also optionally contains the index number of another entPhysicalEntry indicating a containment relationship between the two.

The entityLogical group describes the logical entities managed by a single agent. This group contains a single table to identify logical entities, called the entLogicalTable. The entLogicalTable contains one row per logical entity.  Each row is indexed by an arbitrary, small integer and contains a name, description, and type of the logical entity. It also contains information to allow access to the MIB information for the logical entity. This includes SNMP versions that use a community name (with some form of implied context representation) and SNMP versions that use the SNMP ARCH [RFC2571] method of context identification. If a agent represents multiple logical entities with this MIB, then this group must be implemented for all logical entities known to the agent. If an agent represents a single logical entity, or multiple logical entities within a single naming scope, then implementation of this group may be omitted by the agent.

The entityMapping group describes the associations between the physical entities logical entities, interfaces, and non-interface ports managed by a single agent. This group contains three tables to identify associations between different system components. The entLPMappingTable contains mappings between entLogicalIndex values (logical entities) and entPhysicalIndex values (the physical components supporting that entity). A logical entity can map to more than one physical component, and more than one logical entity can map to (share) the same physical component.  If an agent represents a single logical entity, or multiple logical entities within a single naming scope, then implementation of this table may be omitted by the agent. The entAliasMappingTable contains mappings between entLogicalIndex, entPhysicalIndex pairs and 'alias' object identifier values.  This allows resources managed with other MIBs (e.g., repeater ports, bridge ports, physical and logical interfaces) to be identified in the physical entity hierarchy. Note that each alias identifier isonly relevant in a particular naming scope.  If an agent represents a single logical entity, or multiple logical entities within a single naming scope, then implementation of this table may be omitted by the agent. The entPhysicalContainsTable contains simple mappings between 'entPhysicalContainedIn' values for each container/'containee' relationship in the managed system. The indexing of this table allows an NMS to quickly discover the 'entPhysicalIndex' values for all children of a given physical entity.

The entityGeneral group describes general system attributes shared by potentially all types of entities managed by a single agent. This group contains general information relating to the other object groups. At this time, the entGeneral group contains a single scalar object (entLastChangeTime), which represents the value of sysUptime when any part of the Entity MIB configuration last changed.

The entityNotifications group contains status indication notifications. This group contains notification definitions relating to the overall status of the Entity MIB instantiation.

The following table shows the index range of each physical class of each stack in S5150EI products. It supports 10 units in a stack at max. There are 1 slot, 1~3 PSU, 1~5 Fans and 1~9 Sensors as level 1 containers, and 2 sub slots as level 2 containers in one unit. There are front panel sub slot, stacking ports sub slot, expansion card sub slots. The max port number in a slot is 52. It also support 1 out-band management port. Such as the figure and the table shown:

403

373

Sslt

Card

464~515

Port

Stack

Port

412~463

193

194

195

196

197

198

199

200

201

202

203

204

205

206

Sslt

Card

372

402

Slt

27

sensor

Slt

sensor

28

207

208

Slt

sensor

209

29

Unit

Unit

2

3

Unit

Unit

4

5

Unit

Unit

6

7

Unit

Unit

8

9

Unit

10

11

Slt

12

brd

Slt

13

pwr

Slt

pwr

14

Slt

15

rps

Slt

fan

16

Slt

17

fan

Slt

fan

18

Slt

19

fan

Slt

fan

20

Slt

21

sensor

Slt

sensor

22

Slt

23

sensor

Slt

sensor

24

Slt

25

sensor

Slt

sensor

26

192

1

Unit

Sslt

Card

Port

374

404

516~567

| User sees | entPhysicalIndex | entPhysicalClass | entPhysicalContainedIn | entPhysicalParentRelPos |
| --- | --- | --- | --- | --- |
| Stack | 1 | Stack (11) | 0 | -1 |
| Unit1~unit10 | 2~11 | Chasis(3) | 1 | 1~10 |
| Slot0(for board0) | 12 | container(5) | 2 | 1 |
| slot1(for PWR1) | 13 | container(5) | 2 | 2 |
| Slot2(for PWR2) | 14 | container(5) | 2 | 3 |
| Slot3(for RPS) | 15 | container(5) | 2 | 4 |
| Slot4(for FAN1) | 16 | container(5) | 2 | 5 |
| Slot5(for FAN2) | 17 | container(5) | 2 | 6 |
| Slot6(for FAN3) | 18 | container(5) | 2 | 7 |
| Slot7(for FAN4) | 19 | container(5) | 2 | 8 |
| Slot8(for FAN5) | 20 | container(5) | 2 | 9 |
| Slot9(for senser1) | 21 | container(5) | 2 | 10 |
| Slot10(for senser2) | 22 | container(5) | 2 | 11 |
| Slot11(for senser3) | 23 | container(5) | 2 | 12 |
| Slot12(for senser4) | 24 | container(5) | 2 | 13 |
| Slot13(for senser5) | 25 | container(5) | 2 | 14 |
| Slot14(for senser6) | 26 | container(5) | 2 | 15 |
| Slot15(for senser7) | 27 | container(5) | 2 | 16 |
| Slot16(for senser8) | 28 | container(5) | 2 | 17 |
| Slot17(for senser9) | 29 | container(5) | 2 | 18 |
| Board0(a virtual board that contains subslots) | 192 | module(9) | 12 | 1 |
| PWR1 | 193 | powerSupply(6) | 13 | 1 |
| PWR2 | 194 | powerSupply(6) | 14 | 1 |
| RPS1 | 195 | powerSupply(6) | 15 | 1 |
| FAN1 | 196 | fan(7) | 16 | 1 |
| FAN2 | 197 | fan(7) | 17 | 1 |
| FAN3 | 198 | fan(7) | 18 | 1 |
| FAN4 | 199 | fan(7) | 19 | 1 |
| FAN5 | 200 | fan(7) | 20 | 1 |
| Sensor1 | 201 | Sensor(8) | 21 | 1 |
| Sensor2 | 202 | Sensor(8) | 22 | 1 |
| Sensor3 | 203 | Sensor(8) | 23 | 1 |
| Sensor4 | 204 | Sensor(8) | 24 | 1 |
| Sensor5 | 205 | Sensor(8) | 25 | 1 |
| Sensor6 | 206 | Sensor(8) | 26 | 1 |
| Sensor7 | 207 | Sensor(8) | 27 | 1 |
| Sensor8 | 208 | Sensor(8) | 28 | 1 |
| Sensor9 | 209 | Sensor(8) | 29 | 1 |
| SubSlot0 | 372 | container(5) | 192 | 1 |
| SubSlot1 | 373 | container(5) | 192 | 1 |
| SubSlot2 | 374 | container(5) | 192 | 1 |
| SubCard0(Front Panel) | 402 | module(9) | 372 | 1 |
| SubCard1(Expan Card 1) | 403 | module(9) | 373 | 1 |
| SubCard2(Expan Card 2) | 404 | module(9) | 374 | 1 |
| Pt1~Pt52on Scard0 | 412~463 | port(10) | 402 | 1~52 |
| Pt1~Pt52on Scard1 | 464~515 | port(10) | 403 | 1~52 |
| Pt1~Pt52on Scard2 | 516~567 | port(10) | 404 | 1~52 |
| Fabric | 10001 | stack(11) | 0 | -1 |

entPhysicalTable

OID of this table is: 1.3.6.1.2.1.47.1.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entPhysicalIndex (1.3.6.1.2.1.47.1.1.1.1.1) | not-accessible | No | As per MIB |
| entPhysicalDescr (1.3.6.1.2.1.47.1.1.1.1.2) | read-only | No | As per MIB |
| entPhysicalVendorType (1.3.6.1.2.1.47.1.1.1.1.3) | read-only | No | All the vendor types are listed in HH3C-ENTITY-VENDORTYPE-OID-MIB. When provided for different company, the value of this object should be described separately. |
| entPhysicalContainedIn (1.3.6.1.2.1.47.1.1.1.1.4) | read-only | No | As per MIB |
| entPhysicalClass (1.3.6.1.2.1.47.1.1.1.1.5) | read-only | No | As per MIB |
| entPhysicalParentRelPos (1.3.6.1.2.1.47.1.1.1.1.6) | read-only | No | As per MIB |
| entPhysicalName (1.3.6.1.2.1.47.1.1.1.1.7) | read-only | No | As per MIB |
| entPhysicalHardwareRev (1.3.6.1.2.1.47.1.1.1.1.8) | read-only | No | As per MIB |
| entPhysicalFirmwareRev (1.3.6.1.2.1.47.1.1.1.1.9) | read-only | No | As per MIB |
| entPhysicalSoftwareRev (1.3.6.1.2.1.47.1.1.1.1.10) | read-only | No | As per MIB |
| entPhysicalSerialNum (1.3.6.1.2.1.47.1.1.1.1.11) | read-write | No | Only support read operation |
| entPhysicalMfgName (1.3.6.1.2.1.47.1.1.1.1.12) | read-only | No | As per MIB |
| entPhysicalModelName (1.3.6.1.2.1.47.1.1.1.1.13) | read-only | No | As per MIB |
| entPhysicalAlias (1.3.6.1.2.1.47.1.1.1.1.14) | read-write | No | Only support read operation |
| entPhysicalAssetID (1.3.6.1.2.1.47.1.1.1.1.15) | read-write | No | Only support read operation |
| entPhysicalIsFRU (1.3.6.1.2.1.47.1.1.1.1.16) | read-only | No | As per MIB |
| entPhysicalMfgDate (1.3.6.1.2.1.47.1.1.1.1.17) | read-only | No | As per MIB |
| entPhysicalUris (1.3.6.1.2.1.47.1.1.1.1.18) | read-write | No | Only support read operation |

entPhysicalIsFRU

| entity | entPhysicalIsFRU |
| --- | --- |
| Unit | True(1) |
| Slot | False(2) |
| SubSlot | False(2) |
| PowerSupply | True(1) |
| Fan | True(1) |
| Board | False(2) |
| SubCard | True(1) |
| Port | If an optical module port, True(1); else, False(2) |
| Fabric | False(2) |

- entLogicalTable

OID of this table is: 1.3.6.1.2.1.47.1.2.1

This table is not supported.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entLogicalIndex (1.3.6.1.2.1.47.1.2.1.1.1) | not-accessible | No | As per MIB |
| entLogicalDescr (1.3.6.1.2.1.47.1.2.1.1.2) | read-only | No | As per MIB |
| entLogicalType (1.3.6.1.2.1.47.1.2.1.1.3) | read-only | No | As per MIB |
| entLogicalCommunity (1.3.6.1.2.1.47.1.2.1.1.4) | read-only | No | As per MIB |
| entLogicalTAddress (1.3.6.1.2.1.47.1.2.1.1.5) | read-only | No | As per MIB |
| entLogicalTDomain (1.3.6.1.2.1.47.1.2.1.1.6) | read-only | No | As per MIB |
| entLogicalContextEngineID (1.3.6.1.2.1.47.1.2.1.1.7) | read-only | No | As per MIB |
| entLogicalContextName (1.3.6.1.2.1.47.1.2.1.1.8) | read-only | No | As per MIB |

- entLPMappingTable

OID of this table is: 1.3.6.1.2.1.47.1.3.1

This table is not supported.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entLPPhysicalIndex (1.3.6.1.2.1.47.1.3.1.1.1) | read-only | No | As per MIB |

- entAliasMappingTable

OID of this table is: 1.3.6.1.2.1.47.1.3.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entAliasLogicalIndexOrZero (1.3.6.1.2.1.47.1.3.2.1.1) | not-accessible | No | As per MIB |
| entAliasMappingIdentifier (1.3.6.1.2.1.47.1.3.2.1.2) | read-only | No | As per MIB |

- entPhysicalContainsTable

OID of this table is: 1.3.6.1.2.1.47.1.3.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entPhysicalChildIndex (1.3.6.1.2.1.47.1.3.3.1.1) | read-only | No | As per MIB |

- entityGeneral Group

OID of this table is: 1.3.6.1.2.1.47.1.4

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| entLastChangeTime (1.3.6.1.2.1.47.1.4.1) | read-only | No | As per MIB |