# Contents

HH3C-USER-MIB	2

Scalar Objects	2

hh3cUserInfoTable	2

hh3cUserAttributeTable	3

hh3cUserRoleTable	4

hh3cUserGroupInfoTable	4

HH3C-USER-MIB

The HH3C-USER-MIB is used to manage local users of network class.

Scalar Objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cUserMaxNum (1.3.6.1.4.1.25506.2.12.1.3) | read-only | No | As per MIB |
| hh3cUserCurrNum (1.3.6.1.4.1.25506.2.12.1.4) | read-only | Current | As per MIB |
| hh3cUserIndexIndicator (1.3.6.1.4.1.25506.2.12.1.5) | read-only | Current | As per MIB |

hh3cUserInfoTable

OID of this table is: 1.3.6.1.4.1.25506.2.12.1.1

If hh3cUserPassword and hh3cAuthMode would be set, they must be set simultaneously.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cUserName (1.3.6.1.4.1.25506.2.12.1.1.1.1) | read-create | Current | The value could not be null, and could not be “a”, “al” and “all”. The value could not contain some characters, such as ‘\’, ‘/’, ‘:’, ‘*’, ‘?’, ‘<’, ‘>’ ,‘¦’ and’@’ etc. The length of the value must no more than 55. The user name which will be created must not be existed, and it is case sensitive. Can not be modified after creation. |
| hh3cUserPassword (1.3.6.1.4.1.25506.2.12.1.1.1.2) | read-create | Current | A cipher password is a case-sensitive string of 1 to 117 characters. A plaintext password is a case-sensitive string of 1 to 63 characters. Get operation will return null string |
| hh3cAuthMode (1.3.6.1.4.1.25506.2.12.1.1.1.3) | read-create | Current | The value must be 0 or 7. |
| hh3cUserLevel (1.3.6.1.4.1.25506.2.12.1.1.1.4) | read-create | Current | Not supported |
| hh3cUserState (1.3.6.1.4.1.25506.2.12.1.1.1.5) | read-create | Current | As per MIB |
| hh3cUserInfoRowStatus (1.3.6.1.4.1.25506.2.12.1.1.1.6) | read-create | Current | As per MIB |
| hh3cUserIndex (1.3.6.1.4.1.25506.2.12.1.1.1.7) | not-accessible | Current | Range from 1 to 1024. The value can’t be saved. |

hh3cUserAttributeTable

OID of this table is: 1.3.6.1.4.1.25506.2.12.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cAccessLimit (1.3.6.1.4.1.25506.2.12.1.2.1.1) | read-write | Current | The maximum user number of current user who can access devices. Default is 0, means no limit. Range from 0 to 1024. |
| hh3cIdleCut (1.3.6.1.4.1.25506.2.12.1.2.1.2) | read-write | Current | As per MIB |
| hh3cIPAddress (1.3.6.1.4.1.25506.2.12.1.2.1.3) | read-write | Current | Set local user's ip address. The default value is 0.0.0.0. |
| hh3cNasIPAddress (1.3.6.1.4.1.25506.2.12.1.2.1.4) | read-write | Current | Not supported |
| hh3cSlotNum (1.3.6.1.4.1.25506.2.12.1.2.1.5) | read-write | Current | Not supported |
| hh3cSubSlotNum (1.3.6.1.4.1.25506.2.12.1.2.1.6) | read-write | Current | Not supported |
| hh3cPortNum (1.3.6.1.4.1.25506.2.12.1.2.1.7) | read-write | Current | Not supported |
| hh3cMacAddress (1.3.6.1.4.1.25506.2.12.1.2.1.8) | read-write | Current | As per MIB |
| hh3cVlan (1.3.6.1.4.1.25506.2.12.1.2.1.9) | read-write | Current | As per MIB |
| hh3cFtpService (1.3.6.1.4.1.25506.2.12.1.2.1.10) | read-write | Current | Not supported |
| hh3cFtpDirectory (1.3.6.1.4.1.25506.2.12.1.2.1.11) | read-write | Current | Not supported |
| hh3cLanAccessService (1.3.6.1.4.1.25506.2.12.1.2.1.12) | read-write | Current | As per MIB |
| hh3cSshService (1.3.6.1.4.1.25506.2.12.1.2.1.13) | read-write | Current | Not supported |
| hh3cTelnetService (1.3.6.1.4.1.25506.2.12.1.2.1.14) | read-write | Current | Not supported |
| hh3cTerminalService (1.3.6.1.4.1.25506.2.12.1.2.1.15) | read-write | Current | Not supported |
| hh3cExpirationDate (1.3.6.1.4.1.25506.2.12.1.2.1.16) | read-write | Current | Not supported |
| hh3cUserGroup (1.3.6.1.4.1.25506.2.12.1.2.1.17) | read-write | Current | The maximum length of the group name is 32. |
| hh3cPortalService (1.3.6.1.4.1.25506.2.12.1.2.1.18) | read-write | Current | As per MIB |
| hh3cPPPService (1.3.6.1.4.1.25506.2.12.1.2.1.19) | read-write | Current | As per MIB |
| hh3cHttpService (1.3.6.1.4.1.25506.2.12.1.2.1.20) | read-write | Current | Not supported |
| hh3cHttpsService (1.3.6.1.4.1.25506.2.12.1.2.1.21) | read-write | Current | Not supported |
| hh3cUserIfIndex (1.3.6.1.4.1.25506.2.12.1.2.1.22) | read-write | Current | As per MIB |

hh3cUserRoleTable

OID of this table is: 1.3.6.1.4.1.25506.2.12.1.6

hh3cUserRoleTable is local user role table. If there are data in hh3cUserInfoTable, this table should have the relevant data.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cUserRole (1.3.6.1.4.1.25506.2.12.1.6.1.1) | not-accessable | Current | The default role is network-operater or mdc-operator. The maximum length of the role name is 63 |
| hh3cUserRoleStatus (1.3.6.1.4.1.25506.2.12.1.6.1.2) | read-create | Current | This object is responsible for managing the creation, deletion and modification of rows, which support active status and CreateAndGo, Destroy operation. To create a new row, hh3cUserIndex must be specified. The number of rows with same hh3cUserIndex cann't be more than 64. If the the number of rows with a hh3cUserIndex is 1, this row cann't be destroied. |

hh3cUserGroupInfoTable

OID of this table is: 1.3.6.1.4.1.25506.2.12.2.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| hh3cUserGroupName (1.3.6.1.4.1.25506.2.12.2.1.1.1) | not-accessible | Current | As per MIB |
| hh3cUserGroupInfoRowStatus (1.3.6.1.4.1.25506.2.12.2.1.1.2) | read-create | Current | As per MIB |