# Contents

IPV6-MLD-MIB	2

mldInterfaceTable	2

mldCacheTable	3

IPV6-MLD-MIB

The IPV6-MLD-MIB contains two tables:

(1)  The MLD Interface Table, which contains one row for each interface on which MLD is enabled.

(2)  The MLD Cache Table which contains one row for each IPv6 Multicast group for which there are members on a particular interface.

Both tables are intended to be implemented by hosts and routers. Some objects in each table apply to routers only.

mldInterfaceTable

OID of this table is: 1.3.6.1.2.1.91.1.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mldInterfaceIfIndex (1.3.6.1.2.1.91.1.1.1.1) | not-accessible | Current | As per MIB |
| mldInterfaceQueryInterval (1.3.6.1.2.1.91.1.1.1.2) | read-create | Current | Range from 1 to 31744 |
| mldInterfaceStatus (1.3.6.1.2.1.91.1.1.1.3) | read-create | Current | Only support active(1), createAndGo(4), destroy(6) |
| mldInterfaceVersion (1.3.6.1.2.1.91.1.1.1.4) | read-create | Current | Only support 1 |
| mldInterfaceQuerier (1.3.6.1.2.1.91.1.1.1.5) | read-only | No | As per MIB |
| mldInterfaceQueryMaxResponseDelay (1.3.6.1.2.1.91.1.1.1.6) | read-create | Current | Range from 1 to 3174 |
| mldInterfaceJoins (1.3.6.1.2.1.91.1.1.1.7) | read-only | No | As per MIB |
| mldInterfaceGroups (1.3.6.1.2.1.91.1.1.1.8) | read-only | No | As per MIB |
| mldInterfaceRobustness (1.3.6.1.2.1.91.1.1.1.9) | read-create | Current | Range from 1 to 255 |
| mldInterfaceLastListenQueryIntvl (1.3.6.1.2.1.91.1.1.1.10) | read-create | Current | Range from 1 to 25 |
| mldInterfaceProxyIfIndex (1.3.6.1.2.1.91.1.1.1.11) | read-create | Current | Only support read operation |
| mldInterfaceQuerierUpTime (1.3.6.1.2.1.91.1.1.1.12) | read-only | No | As per MIB |
| mldInterfaceQuerierExpiryTime (1.3.6.1.2.1.91.1.1.1.13) | read-only | No | As per MIB |

mldCacheTable

OID of this table is: 1.3.6.1.2.1.91.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| mldCacheAddress (1.3.6.1.2.1.91.1.2.1.1) | not-accessible | No | As per MIB |
| mldCacheIfIndex (1.3.6.1.2.1.91.1.2.1.2) | not-accessible | No | As per MIB |
| mldCacheSelf (1.3.6.1.2.1.91.1.2.1.3) | read-create | No | Only support read operation, and the default value is false(2) |
| mldCacheLastReporter (1.3.6.1.2.1.91.1.2.1.4) | read-only | No | As per MIB |
| mldCacheUpTime (1.3.6.1.2.1.91.1.2.1.5) | read-only | No | As per MIB |
| mldCacheExpiryTime (1.3.6.1.2.1.91.1.2.1.6) | read-only | No | As per MIB |
| mldCacheStatus (1.3.6.1.2.1.91.1.2.1.7) | read-create | Current | Only support read operation |