# Contents

RFC1213-MIB	2

system Group {mib-2.1}	2

interfaces Group {mib-2.2}	3

at Group {mib-2.3}	3

atTable	3

ip Group {mib-2.4}	4

Scalar Objects	4

ipAddrTable	5

ipRouteTable	5

ipNetToMediaTable	6

ipv6InterfaceTable	6

ipSystemStatsTable	7

ipIfStatsTable	9

ipAddressPrefixTable	11

ipAddressTable	12

ipNetToPhysicalTable	12

ipDefaultRouterTable	13

ipv6RouterAdvertTable	13

icmp Group {mib-2.5}	14

Scalar objects	14

icmpStatsTable	15

icmpMsgStatsTable	15

tcp Group {mib-2.6}	16

Scalar objects	16

tcpConnTable	16

udp Group {mib-2.7}	17

Scalar objects	17

udpTable	17

snmp Group {mib-2.11}	17

RFC1213-MIB

system Group {mib-2.1}

This group is fully supported.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| sysDescr (1.3.6.1.2.1.1.1) | read-only | No | "H3C S5130S" for H3C S5130S series devices; "H3C S3100V3" for H3C S3100V3 series devices |
| sysObjectID (1.3.6.1.2.1.1.2) | read-only | No | H3C S5130S sysoid H3C S3100V3 sysoid |
| sysUpTime (1.3.6.1.2.1.1.3) | read-only | No | As per MIB |
| sysContact (1.3.6.1.2.1.1.4) | read-write | Current | "Hangzhou H3C Technologies Co., Ltd." for H3C devices; "" for HP devices The length of value is 1 to 255. |
| sysName (1.3.6.1.2.1.1.5) | read-write | Current | "H3C" for all H3C brand devices |
| sysLocation (1.3.6.1.2.1.1.6) | read-write | Current | "Hangzhou, China" for H3C devices; "" for HP devices The length of value is 1 to 255. |
| sysServices (1.3.6.1.2.1.1.7) | read-only | Current | 78 (2^ (2-1) + 2^ (3-1) + 2^ (4-1) + 2^ (7-1)). indicates that system can provide the following services: 2 datalink/subnetwork (e.g., bridges) 3 internet (e.g., supports the IP) 4 end-to-end (e.g., supports the TCP) 7 applications (e.g., supports the SMTP) |

interfaces Group {mib-2.2}

See ifTable in IF-MIB

at Group {mib-2.3}

at (Address Translation) group is fully supported.

atTable

OID of this table is: 1.3.6.1.2.1.3.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| atIfIndex (1.3.6.1.2.1.3.1.1.1) | read-write | Current | Only support read operation |
| atPhysAddress (1.3.6.1.2.1.3.1.1.2) | read-write | Current | Only support read operation |
| atNetAddress (1.3.6.1.2.1.3.1.1.3) | read-write | Current | Only support read operation |

ip Group {mib-2.4}

The following objects will be supported:

Scalar Objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipForwarding (1.3.6.1.2.1.4.1) | read-write | Current | Because the L3 switch can not be set to L2 mode via the MIB, so it is a read only object. |
| ipDefaultTTL (1.3.6.1.2.1.4.2) | read-write | No | The number may be set from 1 to 255. |
| ipInReceives (1.3.6.1.2.1.4.3) | read-only | No | As per MIB |
| ipInHdrErrors (1.3.6.1.2.1.4.4) | read-only | No | As per MIB |
| ipInAddrErrors (1.3.6.1.2.1.4.5) | read-only | No | As per MIB |
| ipForwDatagrams (1.3.6.1.2.1.4.6) | read-only | No | As per MIB |
| ipInUnknownProtos (1.3.6.1.2.1.4.7) | read-only | No | As per MIB |
| ipInDiscards (1.3.6.1.2.1.4.8) | read-only | No | As per MIB |
| ipInDelivers (1.3.6.1.2.1.4.9) | read-only | No | As per MIB |
| ipOutRequests (1.3.6.1.2.1.4.10) | read-only | No | As per MIB |
| ipOutDiscards (1.3.6.1.2.1.4.11) | read-only | No | As per MIB |
| ipOutNoRoutes (1.3.6.1.2.1.4.12) | read-only | No | As per MIB |
| ipReasmTimeout (1.3.6.1.2.1.4.13) | read-only | No | As per MIB |
| ipReasmReqds (1.3.6.1.2.1.4.14) | read-only | No | As per MIB |
| ipReasmOKs (1.3.6.1.2.1.4.15) | read-only | No | As per MIB |
| ipReasmFails (1.3.6.1.2.1.4.16) | read-only | No | As per MIB |
| ipFragOKs (1.3.6.1.2.1.4.17) | read-only | No | As per MIB |
| ipFragFails (1.3.6.1.2.1.4.18) | read-only | No | As per MIB |
| ipFragCreates (1.3.6.1.2.1.4.19) | read-only | No | As per MIB |
| ipRoutingDiscards (1.3.6.1.2.1.4.23) | read-only | No | As per MIB |
| ipv6IpForwarding (1.3.6.1.2.1.4.25) | read-write | No | Only support read operation |
| ipv6IpDefaultHopLimit (1.3.6.1.2.1.4.26) | read-write | No | Only support read operation |
| ipv4InterfaceTableLastChange (1.3.6.1.2.1.4.27) | read-only | No | Not supported |
| ipv6InterfaceTableLastChange (1.3.6.1.2.1.4.29) | read-only | No | Not supported |
| ipIfStatsTableLastChange (1.3.6.1.2.1.4.31.2) | read-only | No | Not supported |
| ipAddressSpinLock (1.3.6.1.2.1.4.33) | read-write | No | Not supported |
| ipv6RouterAdvertSpinLock (1.3.6.1.2.1.4.38) | read-write | No | Not supported |

ipAddrTable

OID of this table is: 1.3.6.1.2.1.4.20

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipAdEntAddr (1.3.6.1.2.1.4.20.1.1) | read-only | Current | As per MIB |
| ipAdEntIfIndex (1.3.6.1.2.1.4.20.1.2) | read-only | Current | As per MIB |
| ipAdEntNetMask (1.3.6.1.2.1.4.20.1.3) | read-only | Current | As per MIB |
| ipAdEntBcastAddr (1.3.6.1.2.1.4.20.1.4) | read-only | Current | As per MIB |
| ipAdEntReasmMaxSize (1.3.6.1.2.1.4.20.1.5) | read-only | Current | As per MIB |

ipRouteTable

OID of this table is: 1.3.6.1.2.1.4.21

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipRouteDest(1.3.6.1.2.1.4.21.1.1) | read-write | No | Only support read |
| ipRouteIfIndex(1.3.6.1.2.1.4.21.1.2) | read-write | No | Only support read |
| ipRouteMetric1(1.3.6.1.2.1.4.21.1.3) | read-write | No | Only support read |
| ipRouteMetric2(1.3.6.1.2.1.4.21.1.4) | read-write | No | Only support read |
| ipRouteMetric3(1.3.6.1.2.1.4.21.1.5) | read-write | No | Only support read |
| ipRouteMetric4(1.3.6.1.2.1.4.21.1.6) | read-write | No | Only support read |
| ipRouteNextHop(1.3.6.1.2.1.4.21.1.7) | read-write | No | Only support read |
| ipRouteType(1.3.6.1.2.1.4.21.1.8) | read-write | No | Only support read |
| ipRouteProto(1.3.6.1.2.1.4.21.1.9) | read-only | No | As per MIB |
| ipRouteAge(1.3.6.1.2.1.4.21.1.10) | read-write | No | Only support read |
| ipRouteMask(1.3.6.1.2.1.4.21.1.11) | read-write | No | Only support read |
| ipRouteMetric5(1.3.6.1.2.1.4.21.1.12) | read-write | No | Only support read |
| ipRouteInfo(1.3.6.1.2.1.4.21.1.13) | read-only | No | As per MIB |

ipNetToMediaTable

OID of this table is: 1.3.6.1.2.1.4.22

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipNetToMediaIfIndex (1.3.6.1.2.1.4.22.1.1) | read-write | Current | Only support read operation. |
| ipNetToMediaPhysAddress (1.3.6.1.2.1.4.22.1.2) | read-write | Current | Only support read operation |
| ipNetToMediaNetAddress (1.3.6.1.2.1.4.22.1.3) | read-write | Current | Only support read operation |
| ipNetToMediaType (1.3.6.1.2.1.4.22.1.4) | read-write | Current | Only support read operation |

ipv6InterfaceTable

OID of this table is: 1.3.6.1.2.1.4.30

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipv6InterfaceIfIndex (1.3.6.1.2.1.4.30.1.1) | not-accessible | No | As per MIB |
| ipv6InterfaceReasmMaxSize (1.3.6.1.2.1.4.30.1.2) | read-only | No | As per MIB |
| ipv6InterfaceIdentifier (1.3.6.1.2.1.4.30.1.3) | read-only | No | As per MIB |
| ipv6InterfaceEnableStatus (1.3.6.1.2.1.4.30.1.5) | read-write | No | Only support read operation |
| ipv6InterfaceReachableTime (1.3.6.1.2.1.4.30.1.6) | read-only | No | As per MIB |
| ipv6InterfaceRetransmitTime (1.3.6.1.2.1.4.30.1.7) | read-only | No | As per MIB |
| ipv6InterfaceForwarding (1.3.6.1.2.1.4.30.1.8) | read-write | No | Only support read operation |

ipSystemStatsTable

OID of this table is: 1.3.6.1.2.1.4.31.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipSystemStatsIPVersion (1.3.6.1.2.1.4.31.1.1.1) | not-accessible | No | Only support ipv6(2) |
| ipSystemStatsInReceives (1.3.6.1.2.1.4.31.1.1.3) | read-only | No | As per MIB |
| ipSystemStatsHCInReceives (1.3.6.1.2.1.4.31.1.1.4) | read-only | No | As per MIB |
| ipSystemStatsInOctets (1.3.6.1.2.1.4.31.1.1.5) | read-only | No | Not supported |
| ipSystemStatsHCInOctets (1.3.6.1.2.1.4.31.1.1.6) | read-only | No | Not supported |
| ipSystemStatsInHdrErrors (1.3.6.1.2.1.4.31.1.1.7) | read-only | No | As per MIB |
| ipSystemStatsInNoRoutes (1.3.6.1.2.1.4.31.1.1.8) | read-only | No | As per MIB |
| ipSystemStatsInAddrErrors (1.3.6.1.2.1.4.31.1.1.9) | read-only | No | As per MIB |
| ipSystemStatsInUnknownProtos (1.3.6.1.2.1.4.31.1.1.10) | read-only | No | As per MIB |
| ipSystemStatsInTruncatedPkts (1.3.6.1.2.1.4.31.1.1.11) | read-only | No | Not supported |
| ipSystemStatsInForwDatagrams (1.3.6.1.2.1.4.31.1.1.12) | read-only | No | As per MIB |
| ipSystemStatsHCInForwDatagrams (1.3.6.1.2.1.4.31.1.1.13) | read-only | No | As per MIB |
| ipSystemStatsReasmReqds (1.3.6.1.2.1.4.31.1.1.14) | read-only | No | As per MIB |
| ipSystemStatsReasmOKs (1.3.6.1.2.1.4.31.1.1.15) | read-only | No | As per MIB |
| ipSystemStatsReasmFails (1.3.6.1.2.1.4.31.1.1.16) | read-only | No | As per MIB |
| ipSystemStatsInDiscards (1.3.6.1.2.1.4.31.1.1.17) | read-only | No | As per MIB |
| ipSystemStatsInDelivers (1.3.6.1.2.1.4.31.1.1.18) | read-only | No | As per MIB |
| ipSystemStatsHCInDelivers (1.3.6.1.2.1.4.31.1.1.19) | read-only | No | Not supported |
| ipSystemStatsOutRequests (1.3.6.1.2.1.4.31.1.1.20) | read-only | No | As per MIB |
| ipSystemStatsHCOutRequests (1.3.6.1.2.1.4.31.1.1.21) | read-only | No | Not supported |
| ipSystemStatsOutNoRoutes (1.3.6.1.2.1.4.31.1.1.22) | read-only | No | As per MIB |
| ipSystemStatsOutForwDatagrams (1.3.6.1.2.1.4.31.1.1.23) | read-only | No | As per MIB |
| ipSystemStatsHCOutForwDatagrams (1.3.6.1.2.1.4.31.1.1.24) | read-only | No | As per MIB |
| ipSystemStatsOutDiscards (1.3.6.1.2.1.4.31.1.1.25) | read-only | No | As per MIB |
| ipSystemStatsOutFragReqds (1.3.6.1.2.1.4.31.1.1.26) | read-only | No | As per MIB |
| ipSystemStatsOutFragOKs (1.3.6.1.2.1.4.31.1.1.27) | read-only | No | As per MIB |
| ipSystemStatsOutFragFails (1.3.6.1.2.1.4.31.1.1.28) | read-only | No | As per MIB |
| ipSystemStatsOutFragCreates (1.3.6.1.2.1.4.31.1.1.29) | read-only | No | As per MIB |
| ipSystemStatsOutTransmits (1.3.6.1.2.1.4.31.1.1.30) | read-only | No | As per MIB |
| ipSystemStatsHCOutTransmits (1.3.6.1.2.1.4.31.1.1.31) | read-only | No | As per MIB |
| ipSystemStatsOutOctets (1.3.6.1.2.1.4.31.1.1.32) | read-only | No | Not supported |
| ipSystemStatsHCOutOctets (1.3.6.1.2.1.4.31.1.1.33) | read-only | No | Not supported |
| ipSystemStatsInMcastPkts (1.3.6.1.2.1.4.31.1.1.34) | read-only | No | Not supported |
| ipSystemStatsHCInMcastPkts (1.3.6.1.2.1.4.31.1.1.35) | read-only | No | Not supported |
| ipSystemStatsInMcastOctets (1.3.6.1.2.1.4.31.1.1.36) | read-only | No | Not supported |
| ipSystemStatsHCInMcastOctets (1.3.6.1.2.1.4.31.1.1.37) | read-only | No | Not supported |
| ipSystemStatsOutMcastPkts (1.3.6.1.2.1.4.31.1.1.38) | read-only | No | Not supported |
| ipSystemStatsHCOutMcastPkts (1.3.6.1.2.1.4.31.1.1.39) | read-only | No | Not supported |
| ipSystemStatsOutMcastOctets (1.3.6.1.2.1.4.31.1.1.40) | read-only | No | Not supported |
| ipSystemStatsHCOutMcastOctets (1.3.6.1.2.1.4.31.1.1.41) | read-only | No | Not supported |
| ipSystemStatsInBcastPkts (1.3.6.1.2.1.4.31.1.1.42) | read-only | No | Not supported |
| ipSystemStatsHCInBcastPkts (1.3.6.1.2.1.4.31.1.1.43) | read-only | No | Not supported |
| ipSystemStatsOutBcastPkts (1.3.6.1.2.1.4.31.1.1.44) | read-only | No | Not supported |
| ipSystemStatsHCOutBcastPkts (1.3.6.1.2.1.4.31.1.1.45) | read-only | No | Not supported |
| ipSystemStatsDiscontinuityTime (1.3.6.1.2.1.4.31.1.1.46) | read-only | No | Not supported |
| ipSystemStatsRefreshRate (1.3.6.1.2.1.4.31.1.1.47) | read-only | No | Not supported |

ipIfStatsTable

OID of this table is: 1.3.6.1.2.1.4.31.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipIfStatsIPVersion (1.3.6.1.2.1.4.31.3.1.1) | not-accessible | No | Only support ipv6(2) |
| ipIfStatsIfIndex (1.3.6.1.2.1.4.31.3.1.2) | not-accessible | No | As per MIB |
| ipIfStatsInReceives (1.3.6.1.2.1.4.31.3.1.3) | read-only | No | As per MIB |
| ipIfStatsHCInReceives (1.3.6.1.2.1.4.31.3.1.4) | read-only | No | Not supported |
| ipIfStatsInOctets (1.3.6.1.2.1.4.31.3.1.5) | read-only | No | Not supported |
| ipIfStatsHCInOctets (1.3.6.1.2.1.4.31.3.1.6) | read-only | No | Not supported |
| ipIfStatsInHdrErrors (1.3.6.1.2.1.4.31.3.1.7) | read-only | No | As per MIB |
| ipIfStatsInNoRoutes (1.3.6.1.2.1.4.31.3.1.8) | read-only | No | As per MIB |
| ipIfStatsInAddrErrors (1.3.6.1.2.1.4.31.3.1.9) | read-only | No | As per MIB |
| ipIfStatsInUnknownProtos (1.3.6.1.2.1.4.31.3.1.10) | read-only | No | As per MIB |
| ipIfStatsInTruncatedPkts (1.3.6.1.2.1.4.31.3.1.11) | read-only | No | As per MIB |
| ipIfStatsInForwDatagrams (1.3.6.1.2.1.4.31.3.1.12) | read-only | No | As per MIB |
| ipIfStatsHCInForwDatagrams (1.3.6.1.2.1.4.31.3.1.13) | read-only | No | As per MIB |
| ipIfStatsReasmReqds (1.3.6.1.2.1.4.31.3.1.14) | read-only | No | As per MIB |
| ipIfStatsReasmOKs (1.3.6.1.2.1.4.31.3.1.15) | read-only | No | As per MIB |
| ipIfStatsReasmFails (1.3.6.1.2.1.4.31.3.1.16) | read-only | No | As per MIB |
| ipIfStatsInDiscards (1.3.6.1.2.1.4.31.3.1.17) | read-only | No | As per MIB |
| ipIfStatsInDelivers (1.3.6.1.2.1.4.31.3.1.18) | read-only | No | As per MIB |
| ipIfStatsHCInDelivers (1.3.6.1.2.1.4.31.3.1.19) | read-only | No | Not supported |
| ipIfStatsOutRequests (1.3.6.1.2.1.4.31.3.1.20) | read-only | No | As per MIB |
| ipIfStatsHCOutRequests (1.3.6.1.2.1.4.31.3.1.21) | read-only | No | Not supported |
| ipIfStatsOutForwDatagrams (1.3.6.1.2.1.4.31.3.1.23) | read-only | No | As per MIB |
| ipIfStatsHCOutForwDatagrams (1.3.6.1.2.1.4.31.3.1.24) | read-only | No | Not supported |
| ipIfStatsOutDiscards (1.3.6.1.2.1.4.31.3.1.25) | read-only | No | As per MIB |
| ipIfStatsOutFragReqds (1.3.6.1.2.1.4.31.3.1.26) | read-only | No | As per MIB |
| ipIfStatsOutFragOKs (1.3.6.1.2.1.4.31.3.1.27) | read-only | No | As per MIB |
| ipIfStatsOutFragFails (1.3.6.1.2.1.4.31.3.1.28) | read-only | No | As per MIB |
| ipIfStatsOutFragCreates (1.3.6.1.2.1.4.31.3.1.29) | read-only | No | As per MIB |
| ipIfStatsOutTransmits (1.3.6.1.2.1.4.31.3.1.30) | read-only | No | As per MIB |
| ipIfStatsHCOutTransmits (1.3.6.1.2.1.4.31.3.1.31) | read-only | No | Not supported |
| ipIfStatsOutOctets (1.3.6.1.2.1.4.31.3.1.32) | read-only | No | Not supported |
| ipIfStatsHCOutOctets (1.3.6.1.2.1.4.31.3.1.33) | read-only | No | Not supported |
| ipIfStatsInMcastPkts (1.3.6.1.2.1.4.31.3.1.34) | read-only | No | As per MIB |
| ipIfStatsHCInMcastPkts (1.3.6.1.2.1.4.31.3.1.35) | read-only | No | Not supported |
| ipIfStatsInMcastOctets (1.3.6.1.2.1.4.31.3.1.36) | read-only | No | Not supported |
| ipIfStatsHCInMcastOctets (1.3.6.1.2.1.4.31.3.1.37) | read-only | No | Not supported |
| ipIfStatsOutMcastPkts (1.3.6.1.2.1.4.31.3.1.38) | read-only | No | As per MIB |
| ipIfStatsHCOutMcastPkts (1.3.6.1.2.1.4.31.3.1.39) | read-only | No | Not supported |
| ipIfStatsOutMcastOctets (1.3.6.1.2.1.4.31.3.1.40) | read-only | No | Not supported |
| ipIfStatsHCOutMcastOctets (1.3.6.1.2.1.4.31.3.1.41) | read-only | No | Not supported |
| ipIfStatsInBcastPkts (1.3.6.1.2.1.4.31.3.1.42) | read-only | No | Not supported |
| ipIfStatsHCInBcastPkts (1.3.6.1.2.1.4.31.3.1.43) | read-only | No | Not supported |
| ipIfStatsOutBcastPkts (1.3.6.1.2.1.4.31.3.1.44) | read-only | No | Not supported |
| ipIfStatsHCOutBcastPkts (1.3.6.1.2.1.4.31.3.1.45) | read-only | No | Not supported |
| ipIfStatsDiscontinuityTime (1.3.6.1.2.1.4.31.3.1.46) | read-only | No | Not supported |
| ipIfStatsRefreshRate (1.3.6.1.2.1.4.31.3.1.47) | read-only | No | Not supported |

ipAddressPrefixTable

OID of this table is: 1.3.6.1.2.1.4.32

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipAddressPrefixIfIndex (1.3.6.1.2.1.4.32.1.1) | not-accessible | No | As per MIB |
| ipAddressPrefixType (1.3.6.1.2.1.4.32.1.2) | not-accessible | No | Only support ipv6(2) and ipv6z(4) |
| ipAddressPrefixPrefix (1.3.6.1.2.1.4.32.1.3) | not-accessible | No | As per MIB |
| ipAddressPrefixLength (1.3.6.1.2.1.4.32.1.4) | not-accessible | No | As per MIB |
| ipAddressPrefixOrigin (1.3.6.1.2.1.4.32.1.5) | read-only | No | As per MIB |
| ipAddressPrefixOnLinkFlag (1.3.6.1.2.1.4.32.1.6) | read-only | No | As per MIB |
| ipAddressPrefixAutonomousFlag (1.3.6.1.2.1.4.32.1.7) | read-only | No | As per MIB |
| ipAddressPrefixAdvPreferredLifetime (1.3.6.1.2.1.4.32.1.8) | read-only | No | As per MIB |
| ipAddressPrefixAdvValidLifetime (1.3.6.1.2.1.4.32.1.9) | read-only | No | As per MIB |

ipAddressTable

OID of this table is: 1.3.6.1.2.1.4.34

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipAddressAddrType (1.3.6.1.2.1.4.34.1.1) | not-accessible | No | Only support ipv6(2) and ipv6z(4) |
| ipAddressAddr (1.3.6.1.2.1.4.34.1.2) | not-accessible | No | As per MIB |
| ipAddressIfIndex (1.3.6.1.2.1.4.34.1.3) | read-create | No | Only support read operation |
| ipAddressType (1.3.6.1.2.1.4.34.1.4) | read-create | No | Only support read operation |
| ipAddressPrefix (1.3.6.1.2.1.4.34.1.5) | read-only | No | As per MIB |
| ipAddressOrigin (1.3.6.1.2.1.4.34.1.6) | read-only | No | As per MIB |
| ipAddressStatus (1.3.6.1.2.1.4.34.1.7) | read-create | No | Only support read operation |
| ipAddressCreated (1.3.6.1.2.1.4.34.1.8) | read-only | No | As per MIB |
| ipAddressLastChanged (1.3.6.1.2.1.4.34.1.9) | read-only | No | As per MIB |
| ipAddressRowStatus (1.3.6.1.2.1.4.34.1.10) | read-create | No | Only support read operation Only support active(1). |
| ipAddressStorageType (1.3.6.1.2.1.4.34.1.11) | read-only | No | The value is always volatile(2). |

ipNetToPhysicalTable

OID of this table is: 1.3.6.1.2.1.4.35

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipNetToPhysicalIfIndex (1.3.6.1.2.1.4.35.1.1) | not-accessible | No | As per MIB |
| ipNetToPhysicalNetAddressType (1.3.6.1.2.1.4.35.1.2) | not-accessible | No | Only support ipv6(2),ipv6z(4) |
| ipNetToPhysicalNetAddress (1.3.6.1.2.1.4.35.1.3) | not-accessible | No | As per MIB |
| ipNetToPhysicalPhysAddress (1.3.6.1.2.1.4.35.1.4) | read-create | No | Only support read operation |
| ipNetToPhysicalLastUpdated (1.3.6.1.2.1.4.35.1.5) | read-only | No | As per MIB |
| ipNetToPhysicalType (1.3.6.1.2.1.4.35.1.6) | read-create | No | Only support read operation |
| ipNetToPhysicalState (1.3.6.1.2.1.4.35.1.7) | read-only | No | As per MIB |
| ipNetToPhysicalRowStatus (1.3.6.1.2.1.4.35.1.8) | read-create | No | Only support read operation |

ipDefaultRouterTable

OID of this table is: 1.3.6.1.2.1.4.37

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipDefaultRouterAddressType (1.3.6.1.2.1.4.37.1.1) | not-accessible | No | Only support ipv6(2) and ipv6z(4) |
| ipDefaultRouterAddress (1.3.6.1.2.1.4.37.1.2) | not-accessible | No | As per MIB |
| ipDefaultRouterIfIndex (1.3.6.1.2.1.4.37.1.3) | not-accessible | No | As per MIB |
| ipDefaultRouterLifetime (1.3.6.1.2.1.4.37.1.4) | read-only | No | As per MIB |
| ipDefaultRouterPreference (1.3.6.1.2.1.4.37.1.5) | read-only | No | As per MIB |

ipv6RouterAdvertTable

OID of this table is: 1.3.6.1.2.1.4.39

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipv6RouterAdvertIfIndex (1.3.6.1.2.1.4.39.1.1) | not-accessible | No | As per MIB |
| ipv6RouterAdvertSendAdverts (1.3.6.1.2.1.4.39.1.2) | read-create | No | Only support read operation |
| ipv6RouterAdvertMaxInterval (1.3.6.1.2.1.4.39.1.3) | read-create | No | Only support read operation |
| ipv6RouterAdvertMinInterval (1.3.6.1.2.1.4.39.1.4) | read-create | No | Only support read operation |
| ipv6RouterAdvertManagedFlag (1.3.6.1.2.1.4.39.1.5) | read-create | No | Only support read operation |
| ipv6RouterAdvertOtherConfigFlag (1.3.6.1.2.1.4.39.1.6) | read-create | No | Only support read operation |
| ipv6RouterAdvertLinkMTU (1.3.6.1.2.1.4.39.1.7) | read-create | No | Only support read operation |
| ipv6RouterAdvertReachableTime (1.3.6.1.2.1.4.39.1.8) | read-create | No | Only support read operation |
| ipv6RouterAdvertRetransmitTime (1.3.6.1.2.1.4.39.1.9) | read-create | No | Only support read operation |
| ipv6RouterAdvertCurHopLimit (1.3.6.1.2.1.4.39.1.10) | read-create | No | Only support read operation |
| ipv6RouterAdvertDefaultLifetime (1.3.6.1.2.1.4.39.1.11) | read-create | No | Only support read operation |
| ipv6RouterAdvertRowStatus (1.3.6.1.2.1.4.39.1.12) | read-create | No | Only support read operation |

icmp Group {mib-2.5}

Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| icmpInMsgs (1.3.6.1.2.1.5.1) | read-only | No | As per MIB |
| icmpInErrors (1.3.6.1.2.1.5.2) | read-only | No | As per MIB |
| icmpInDestUnreachs (1.3.6.1.2.1.5.3) | read-only | No | As per MIB |
| icmpInTimeExcds (1.3.6.1.2.1.5.4) | read-only | No | As per MIB |
| icmpInParmProbs (1.3.6.1.2.1.5.5) | read-only | No | As per MIB |
| icmpInSrcQuenchs (1.3.6.1.2.1.5.6) | read-only | No | As per MIB |
| icmpInRedirects (1.3.6.1.2.1.5.7) | read-only | No | As per MIB |
| icmpInEchos (1.3.6.1.2.1.5.8) | read-only | No | As per MIB |
| icmpInEchoReps (1.3.6.1.2.1.5.9) | read-only | No | As per MIB |
| icmpInTimestamps (1.3.6.1.2.1.5.10) | read-only | No | As per MIB |
| icmpInTimestampReps (1.3.6.1.2.1.5.11) | read-only | No | As per MIB |
| icmpInAddrMasks (1.3.6.1.2.1.5.12) | read-only | No | As per MIB |
| icmpInAddrMaskReps (1.3.6.1.2.1.5.13) | read-only | No | As per MIB |
| icmpOutMsgs (1.3.6.1.2.1.5.14) | read-only | No | As per MIB |
| icmpOutErrors (1.3.6.1.2.1.5.15) | read-only | No | As per MIB |
| icmpOutDestUnreachs (1.3.6.1.2.1.5.16) | read-only | No | As per MIB |
| icmpOutTimeExcds (1.3.6.1.2.1.5.17) | read-only | No | As per MIB |
| icmpOutParmProbs (1.3.6.1.2.1.5.18) | read-only | No | As per MIB |
| icmpOutSrcQuenchs (1.3.6.1.2.1.5.19) | read-only | No | Not supported |
| icmpOutRedirects (1.3.6.1.2.1.5.20) | read-only | No | As per MIB |
| icmpOutEchos (1.3.6.1.2.1.5.21) | read-only | No | As per MIB |
| icmpOutEchoReps (1.3.6.1.2.1.5.22) | read-only | No | As per MIB |
| icmpOutTimestamps (1.3.6.1.2.1.5.23) | read-only | No | Not supported |
| icmpOutTimestampReps (1.3.6.1.2.1.5.24) | read-only | No | As per MIB |
| icmpOutAddrMasks (1.3.6.1.2.1.5.25) | read-only | No | Not supported |
| icmpOutAddrMaskReps (1.3.6.1.2.1.5.26) | read-only | No | As per MIB |

icmpStatsTable

OID of this table is: 1.3.6.1.2.1.5.29

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| icmpStatsIPVersion | not-accessible | No | Only support ipv6(2) |
| icmpStatsInMsgs | read-only | No | As per MIB |
| icmpStatsInErrors | read-only | No | As per MIB |
| icmpStatsOutMsgs | read-only | No | As per MIB |
| icmpStatsOutErrors | read-only | No | The value is always 0. |

icmpMsgStatsTable

OID of this table is: 1.3.6.1.2.1.5.30

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| icmpMsgStatsIPVersion | not-accessible | No | Only support ipv6(2) |
| icmpMsgStatsType | not-accessible | No | As per MIB |
| icmpMsgStatsInPkts | read-only | No | As per MIB |
| icmpMsgStatsOutPkts | read-only | No | As per MIB |

tcp Group {mib-2.6}

tcp Group in RFC1213-MIB is updated by RFC4022-MIB.

Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| tcpRtoAlgorithm (1.3.6.1.2.1.6.1) | read-only | No | As per MIB |
| tcpRtoMin (1.3.6.1.2.1.6.2) | read-only | No | As per MIB |
| tcpRtoMax (1.3.6.1.2.1.6.3) | read-only | No | As per MIB |
| tcpMaxConn (1.3.6.1.2.1.6.4) | read-only | No | As per MIB |
| tcpActiveOpens (1.3.6.1.2.1.6.5) | read-only | No | As per MIB |
| tcpPassiveOpens (1.3.6.1.2.1.6.6) | read-only | No | As per MIB |
| tcpAttemptFails (1.3.6.1.2.1.6.7) | read-only | No | As per MIB |
| tcpEstabResets (1.3.6.1.2.1.6.8) | read-only | No | As per MIB |
| tcpCurrEstab (1.3.6.1.2.1.6.9) | read-only | No | As per MIB |
| tcpInSegs (1.3.6.1.2.1.6.10) | read-only | No | As per MIB |
| tcpOutSegs (1.3.6.1.2.1.6.11) | read-only | No | As per MIB |
| tcpRetransSegs (1.3.6.1.2.1.6.12) | read-only | No | As per MIB |
| tcpInErrs (1.3.6.1.2.1.6.14) | read-only | No | As per MIB |
| tcpOutRsts (1.3.6.1.2.1.6.15) | read-only | No | As per MIB |

tcpConnTable

OID of this table is:  1.3.6.1.2.1.6.13

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| tcpConnState (1.3.6.1.2.1.6.13.1.1) | read-write | No | Only support read operation. |
| tcpConnLocalAddress (1.3.6.1.2.1.6.13.1.2) | read-only | No | As per MIB |
| tcpConnLocalPort (1.3.6.1.2.1.6.13.1.3) | read-only | No | As per MIB |
| tcpConnRemAddress (1.3.6.1.2.1.6.13.1.4) | read-only | No | As per MIB |
| tcpConnRemPort (1.3.6.1.2.1.6.13.1.5) | read-only | No | As per MIB |

udp Group {mib-2.7}

udp Group in RFC1213-MIB is updated by RFC4113-MIB.

Scalar objects

A statistics collection for UDP group.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| udpInDatagrams (1.3.6.1.2.1.7.1) | read- only | No | As per MIB |
| udpNoPorts (1.3.6.1.2.1.7.2) | read-only | No | As per MIB |
| udpInErrors (1.3.6.1.2.1.7.3) | read-only | No | As per MIB |
| udpOutDatagrams (1.3.6.1.2.1.7.4) | read-only | No | As per MIB |

udpTable

OID of this table is:  1.3.6.1.2.1.7.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| udpLocalAddress (1.3.6.1.2.1.7.5.1.1) | read-only | Current | As per MIB |
| udpLocalPort (1.3.6.1.2.1.7.5.1.2) | read-only | Current | As per MIB |

snmp Group {mib-2.11}

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| snmpInPkts (1.3.6.1.2.1.11.1) | read-only | No | As per MIB |
| snmpOutPkts (1.3.6.1.2.1.11.2) | read-only | No | As per MIB |
| snmpInBadVersions (1.3.6.1.2.1.11.3) | read-only | No | As per MIB |
| snmpInBadCommunityNames (1.3.6.1.2.1.11.4) | read-only | No | As per MIB |
| snmpInBadCommunityUses (1.3.6.1.2.1.11.5) | read-only | No | As per MIB |
| snmpInASNParseErrs (1.3.6.1.2.1.11.6) | read-only | No | As per MIB |
| snmpInTooBigs (1.3.6.1.2.1.11.8) | read-only | No | The value is always 0. |
| snmpInNoSuchNames (1.3.6.1.2.1.11.9) | read-only | No | The value is always 0. |
| snmpInBadValues (1.3.6.1.2.1.11.10) | read-only | No | The value is always 0. |
| snmpInReadOnlys (1.3.6.1.2.1.11.11) | read-only | No | The value is always 0. |
| snmpInGenErrs (1.3.6.1.2.1.11.12) | read-only | No | The value is always 0. |
| snmpInTotalReqVars (1.3.6.1.2.1.11.13) | read-only | No | As per MIB |
| snmpInTotalSetVars (1.3.6.1.2.1.11.14) | read-only | No | As per MIB |
| snmpInGetRequests (1.3.6.1.2.1.11.15) | read-only | No | As per MIB |
| snmpInGetNexts (1.3.6.1.2.1.11.16) | read-only | No | As per MIB |
| snmpInSetRequests (1.3.6.1.2.1.11.17) | read-only | No | As per MIB |
| snmpInGetResponses (1.3.6.1.2.1.11.18) | read-only | No | Not supported |
| snmpInTraps (1.3.6.1.2.1.11.19) | read-only | No | Not supported |
| snmpOutTooBigs (1.3.6.1.2.1.11.20) | read-only | No | As per MIB |
| snmpOutNoSuchNames (1.3.6.1.2.1.11.21) | read-only | No | As per MIB |
| snmpOutBadValues (1.3.6.1.2.1.11.22) | read-only | No | As per MIB |
| snmpOutGenErrs (1.3.6.1.2.1.11.24) | read-only | No | As per MIB |
| snmpOutGetRequests (1.3.6.1.2.1.11.25) | read-only | No | As per MIB |
| snmpOutGetNexts (1.3.6.1.2.1.11.26) | read-only | No | As per MIB |
| snmpOutSetRequests (1.3.6.1.2.1.11.27) | read-only | No | As per MIB |
| snmpOutGetResponses (1.3.6.1.2.1.11.28) | read-only | No | As per MIB |
| snmpOutTraps (1.3.6.1.2.1.11.29) | read-only | No | As per MIB |
| snmpEnableAuthenTraps (1.3.6.1.2.1.11.30) | read-write | Current | The default value is enabled(1). |