# Contents

SNMP-FRAMEWORK-MIB	2

Snmp Framework AdminSnmp Engine Group	2

The allocation algorithm of snmpEngineID	2

The algorithm of snmpEngineBoots	2

SNMP-FRAMEWORK-MIB

Snmp Framework AdminSnmp Engine Group

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| snmpEngineID (1.3.6.1.6.3.10.2.1.1) | read-only | Current | As per MIB |
| snmpEngineBoots (1.3.6.1.6.3.10.2.1.2) | read-only | Yes | See the algorithm of snmpEngineBoots below |
| snmpEngineTime (1.3.6.1.6.3.10.2.1.3) | read-only | No | As per MIB |
| snmpEngineMaxMessageSize (1.3.6.1.6.3.10.2.1.4) | read-only | Current | As per MIB |

The allocation algorithm of snmpEngineID

The snmpEngineID has a length of 16 octets:

The first four octets are set to the binary equivalent of the agent's SNMP management private enterprise number as assigned by the Internet Assigned Numbers Authority (IANA), and the very first bit is set to 1. For example, if H3C Networks has been assigned enterprises 25506, the first four octets would be assigned '800063A2'H.

For routers, the remaining eight octets are random octets.

For switches, the fifth octet is 8 indicates that the following is enterprise-specified. The following six octets are the MAC address. The remaining four octets are the MDC number.

The algorithm of snmpEngineBoots

If SNMP becomes enabled from disabled, then the snmpEngineBoots increments 1.

If the snmpEngineID has been configured and its value differs from the former one, then the snmpEngineBoots is reverted to 1.

If the snmpEngineTime reachs its maximum value then the snmpEngineBoots increments 1.