# Contents

SNMP-TARGET-MIB	2

Scalar objects of snmpTargetObjects group	2

snmpTargetAddrTable	2

snmpTargetParamsTable	3

SNMP-TARGET-MIB

snmpTargetAddrTable and snmpTargetParamsTable combined are used to create a valid target host row in device. The command line corresponding to this entry can be displayed as “snmp-agent target-host trap address udp-domain …”. There are some rules need conformed with.

For a valid target host entry which is made up of one entry in snmpTargetAddrTable and another entry in snmpTargetParamsTable:

The value of snmpTargetAddrName is the same as the value of snmpTargetAddrParams.

The value of snmpTargetAddrParams is the same as the value of snmpTargetParamsName.

The value format of snmpTargetAddrName is something like “traphost.” + snmpTargetParamsSecurityName + IpAddress+ VPN Instance name. The IpAddress is the ip address part of snmpTargetAddrTAddress. If not support VPN，VPN Instance name does not need to be configured.

The value of snmpTargetAddrTagList must be octect string “TrapHost”.

For IPV4, the value of snmpTargetAddrTDomain must be snmpUDPDomain.

The value of snmpTargetAddrStorageType should be nonVolatile.

The length of snmpTargetParamsSecurityName in snmpTargetAddrParams restricted in 1 to 32 octets.

The length of VPN Instance name in snmpTargetAddrParams restricted in 1 to 31 octets.

Scalar objects of snmpTargetObjects group

OID of this table is: 1.3.6.1.6.3.12.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| snmpTargetSpinLock (1.3.6.1.6.3.12.1.1) | read-write | No | As per MIB |
| snmpUnavailableContexts (1.3.6.1.6.3.12.1.4) | read-only | No | As per MIB |
| snmpUnknownContexts (1.3.6.1.6.3.12.1.5) | read-only | No | As per MIB |

snmpTargetAddrTable

OID of this table is: 1.3.6.1.6.3.12.1.2

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| snmpTargetAddrName (1.3.6.1.6.3.12.1.2.1.1) | not-accessible | Current | The length of snmpTargetAddrName is from 1 to 255. The ASCII value of the characters of this object should be from 33 to 126 except 63(‘?’). |
| snmpTargetAddrTDomain (1.3.6.1.6.3.12.1.2.1.2) | read-create | Current | This object should be snmpUDPDomain |
| snmpTargetAddrTAddress (1.3.6.1.6.3.12.1.2.1.3) | read-create | Current | The IpAddress should be the ip address part of snmpTargetAddrTAddress |
| snmpTargetAddrTimeout (1.3.6.1.6.3.12.1.2.1.4) | read-create | No | As per MIB |
| snmpTargetAddrRetryCount (1.3.6.1.6.3.12.1.2.1.5) | read-create | No | As per MIB |
| snmpTargetAddrTagList (1.3.6.1.6.3.12.1.2.1.6) | read-create | Current | The ASCII value of the characters of this object should be from 33 to 126 except 63(‘?’). The target host is valid only when the value of this object is “TrapHost” or “InformHost”, |
| snmpTargetAddrParams (1.3.6.1.6.3.12.1.2.1.7) | read-create | Current | The length of snmpTargetAddrParams is from 1 to 255. The ASCII value of the characters of this object should be from 33 to 126 except 63(‘?’). |
| snmpTargetAddrStorageType (1.3.6.1.6.3.12.1.2.1.8) | read-create | Current | The conceptual row can be created, modified and deleted. It can be modified to other values, but only ‘nonVolatile’ is valid. |
| snmpTargetAddrRowStatus (1.3.6.1.6.3.12.1.2.1.9) | read-create | Current | As per MIB |

snmpTargetParamsTable

OID of this table is: 1.3.6.1.6.3.12.1.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| snmpTargetParamsName (1.3.6.1.6.3.12.1.3.1.1) | not-accessible | Current | The length of snmpTargetParamsName is from 1 to 255. The ASCII value of the characters of this object should be from 33 to 126 except 63(‘?’). |
| snmpTargetParamsMPModel (1.3.6.1.6.3.12.1.3.1.2) | read-create | Current | As per MIB |
| snmpTargetParamsSecurityModel (1.3.6.1.6.3.12.1.3.1.3) | read-create | Current | As per MIB |
| snmpTargetParamsSecurityName (1.3.6.1.6.3.12.1.3.1.4) | read-create | Current | As per MIB |
| snmpTargetParamsSecurityLevel (1.3.6.1.6.3.12.1.3.1.5) | read-create | Current | As per MIB |
| snmpTargetParamsStorageType (1.3.6.1.6.3.12.1.3.1.6) | read-create | Current | The conceptual row can be created, modified and deleted. It can be modified to other values, but only ‘nonVolatile’ is valid. |
| snmpTargetParamsRowStatus (1.3.6.1.6.3.12.1.3.1.7) | read-create | Current | As per MIB |

(1)The following is an example:

***** SNMP QUERY STARTED *****

1: snmpTargetSpinLock.0 (integer) 13736

2: [Loaded: SNMPv2-TM] snmpTargetAddrTDomain.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (object identifier) snmpUDPDomain

3: snmpTargetAddrTAddress.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (octet string) A9.FE.4C.4C.00.A2 (hex)

4: snmpTargetAddrTimeout.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (integer) 1500

5: snmpTargetAddrRetryCount.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (integer) 3

6: snmpTargetAddrTagList.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (octet string) TrapHost [54.72.61.70.48.6F.73.74 (hex)]

7: snmpTargetAddrParams.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (octet string) traphost.gl.169.254.76.76 [74.72.61.70.68.6F.73.74.2E.67.6C.2E.31.36.39.2E.32.35.34.2E.37.36.2E.37.36 (hex)]

8: snmpTargetAddrStorageType.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (integer) nonVolatile(3)

9: snmpTargetAddrRowStatus.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (integer) active(1)

10: snmpTargetParamsMPModel.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (integer) 0

11: snmpTargetParamsSecurityModel.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (integer) 1

12: snmpTargetParamsSecurityName.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (octet string) gl [67.6C (hex)]

13: snmpTargetParamsSecurityLevel.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (integer) noAuthNoPriv(1)

14: snmpTargetParamsStorageType.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (integer) nonVolatile(3)

15: snmpTargetParamsRowStatus.116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54 (integer) active(1)

16: snmpUnavailableContexts.0 (counter) 0

17: snmpUnknownContexts.0 (counter) 0

***** SNMP QUERY FINISHED *****

Above is a retrieve for SNMP TARGET entry.

Here the index of this entry is 116.114.97.112.104.111.115.116.46.103.108.46.49.54.57.46.50.53.52.46.55.54.46.55.54, which is the hex code of string "traphost.gl.169.254.76.76"

The first part, 116.114.97.112.104.111.115.116.46, is hex code of "traphost." string.

The second part, 103.108.46, is ascii code of "gl.".

The last part, 46.49.54.57.46.50.53.52.46.55.54.46.55.54, is hex code of "169.254.76.76".